Components Mappings (Task 1.3b)

for the

Tool for Automating Estimation of DSP Resource Statistics for Waveform Components

Submitted under Subcontract FP-19738-430292

An Integrated Tool for SCA Waveform Development, Testing, and Debugging and a Tool for Automated Estimation of DSP Resource Statistics for Waveform Components

Version 1.1

Revision History

| Version | Summary of Changes | Date |
| --- | --- | --- |
| 0.1 (JN) | Internal Release | 1/31/08 |
| 1.0 (JN) | Initial Release | 3/26/08 |
| 1.1 (JN) | Revised mappings to reflect accounting shift of stack operations from memory to ALU (improves data memory estimate); Viterbi cleaned up Minor typos | 5/15/08 |

# Table of Contents

Table of Contents	2

1	Introduction and Methodology	6

1.1	Overview	6

1.2	Pseudo-code Conventions	7

1.2.1	Use instructions consistent with the least complex DSP	7

1.2.2	Label instruction lines	7

1.2.3	Clearly identify assumptions	7

1.2.4	Register conventions	7

1.2.5	Reset internal settings before exiting	8

1.2.6	Group operations that combine together	8

1.2.7	Attempt to parameterize as much as possible	8

1.2.8	Comment	8

1.3	Creating equations	9

1.4	Modifiers	9

2	Real Filter	12

2.1	Pseudocode	12

2.2	Raw Operations Equation	13

2.3	Impact of Specialized Instructions	13

2.4	Synergistic Modifiers	13

3	Complex Filter	14

3.1	Pseudocode	14

3.2	Raw Operations Equation	16

3.3	Impact of Specialized Instructions	16

3.4	Synergistic Modifiers	16

4	Fast Fourier Transform (FFT)	17

4.1	Pseudocode	17

4.2	Raw Operations Equation	20

4.3	Impact of Specialized Instructions	20

4.4	Synergistic Modifiers	20

5	LMS Equalizer (real)	21

5.1	Pseudocode	21

5.2	Raw Operations Equation	22

5.3	Impact of Specialized Instructions	22

5.4	Synergistic Modifiers	22

6	Taylor Series	23

6.1	Pseudocode	23

6.2	Raw Operations Equation	24

6.3	Impact of Specialized Instructions	24

6.4	Synergistic Modifiers	24

7	CORDIC	25

7.1	Pseudocode	25

7.2	Raw Operations Equation	26

7.3	Impact of Specialized Instructions	26

7.4	Synergistic Modifiers	26

8	Interleaver	27

8.1	Pseudocode	27

8.2	Raw Operations Equation	28

8.3	Impact of Specialized Instructions	28

8.4	Synergistic Modifiers	28

9	DeInterleaver	29

9.1	Pseudocode	29

9.2	Raw Operations Equation	30

9.3	Impact of Specialized Instructions	30

9.4	Synergistic Modifiers	30

10	CIC Filter (Interpolator, M=1)	31

10.1	Pseudocode	31

10.2	Raw Operations Equation	33

10.3	Impact of Specialized Instructions	33

10.4	Synergistic Modifiers	33

11	CRC Encoder	34

11.1	Pseudocode	34

11.2	Raw Operations Equation	35

11.3	Impact of Specialized Instructions	35

11.4	Synergistic Modifiers	35

12	CRC Decoder	36

12.1	Pseudocode	36

12.2	Raw Operations Equation	37

12.3	Impact of Specialized Instructions	37

12.4	Synergistic Modifiers	37

13	Convolutional Encoder	38

13.1	Pseudocode	38

13.2	Raw Operations Equation	39

13.3	Impact of Specialized Instructions	39

13.4	Synergistic Modifiers	39

14	Viterbi Decoder (rate 1/r, hard decisions, traceback = 32)	40

14.1	Pseudocode	40

14.2	Raw Operations	43

14.2.1	Find Max	44

14.2.2	Traceback Unit	44

14.2.3	Add Compare Select	44

14.2.4	Path Metric Unit	44

14.2.5	Hard metric	44

14.2.6	Branch Metric Unit	45

14.2.7	Main Decoder	45

14.2.8	Integrated Equations	45

14.3	Impact of Specialized Instructions and Synergistic	45

14.3.1	Find Max (N-31)	45

14.3.2	Traceback Unit (N-31)	46

14.3.3	Add Compare Select (N*num_states)	46

14.3.4	Path Metric Unit (N)	46

14.3.5	Hard metric (N*num_states*2)	47

14.3.6	Branch Metric Unit (N)	47

14.3.7	Main Decoder	47

14.3.8	Integrated Equations	48

15	Polyphase Interpolator	50

15.1	Pseudocode	50

15.2	Raw Operations	51

15.3	Impact of Specialized Instructions	52

15.4	Synergistic Modifiers	52

16	AM Modulator	53

16.1	Pseudocode	53

16.2	Raw Operations	54

16.3	Impact of Specialized Instructions	54

16.4	Synergistic Modifiers	54

17	AM Demodulator	55

17.1	Pseudocode	55

17.2	Raw Operations	56

17.3	Impact of Specialized Instructions	56

17.4	Synergistic Modifiers	56

18	FM Modulator	57

18.1	Pseudocode	57

18.2	Raw Operations	58

18.3	Impact of Specialized Instructions	58

18.4	Synergistic Modifiers	58

19	FM Demodulator	59

19.1	Pseudocode	59

19.2	Raw Operations	60

19.3	Impact of Specialized Instructions	60

19.4	Synergistic Modifiers	60

20	BPSK Modulator	61

20.1	Pseudocode	61

20.2	Raw Operations	63

20.3	Impact of Specialized Instructions	63

20.4	Synergistic Modifiers	63

21	BPSK Demodulator	64

21.1	Pseudocode	64

21.2	Raw Operations	65

21.3	Impact of Specialized Instructions	65

21.4	Synergistic Modifiers	65

22	BFSK Modulator	66

22.1	Pseudocode	66

22.2	Raw Operations	68

22.3	Impact of Specialized Instructions	68

22.4	Synergistic Modifiers	68

23	BFSK Demodulator	69

23.1	Pseudocode	69

23.2	Raw Operations	70

23.3	Impact of Specialized Instructions	70

23.4	Synergistic Modifiers	70

24	16-QAM Modulator	71

24.1	Pseudocode	71

24.2	Raw Operations	72

24.3	Impact of Specialized Instructions	72

24.4	Synergistic Modifiers	72

25	16-QAM Demodulator	73

25.1	Pseudocode	73

25.2	Raw Operations	74

25.3	Impact of Specialized Instructions	74

25.4	Synergistic Modifiers	74

# Introduction and Methodology

This document is intended to document the steps used to generate the component files created for the “Tool for Automating Estimation of DSP Resource Statistics for Waveform Components” and to provide enough detail for others to be able to create similar files for their components.

This section gives an overview of the methodology for writing a component file and details on key processes associated with this methodology. This is followed by 22 example applications of this process to a variety of different component implementations.

## Overview

A component file is intended to provide a base equation which details all of the instructions necessary to implement a component with the simplest possible instruction set (generally the ARM RISC set). To create this base equation, pseudo-code for the component is written which reflects an anticipated implementation of the component using this instruction set.

In general different DSPs will include specialized instructions and architectural characteristics which permit multiple of these simple instructions to execute in a single cycle. These include instructions that explicitly combine pairs of instructions (e.g., a MAC is a multiplication and an accumulation), ones that obviate the need for instructions (e.g., a block repeat eliminates the need for loop control instructions) and architectural optimizations (e.g., SIMD, VLIW) that permit multiple instructions to be executed in a single cycle. To model these conditions, additional equations are introduced which modify the original base equation.

Note that implementing a component in a different manner (e.g., an algorithmic optimization) will yield different numbers. The advantage of the approach adopted in this project is that rather than having to do a detailed analysis of a component for each DSP, a single component analysis suffices for all mapped DSPs - a significant time savings (e.g., for 20 DSPs, only 1/20th the time is required). Other advantages of this automated process include ensuring that others can leverage the results without having to duplicate the efforts, the separation of DSP analysis from component analysis means that not all systems engineers need to be an expert on all DSPs (each can be an expert on 1 or 2 and write those files), nor do they have to be an expert on all possible waveform components.

A disadvantage to this approach is that it implicitly assumes every instruction takes a single cycle to execute thereby overlooking latency and delay which can vary significantly from DSP to DSP and instruction to instruction. For well-designed pipelined code, this should have minimal impact on estimations as these should fall outside the loop kernel, but is likely the largest a source of error in the estimation method (and means that it should be expected that fewer cycles are reported than would b actually required.

## Pseudo-code Conventions

The first step in estimating the number of cycles required to implement a component is estimating the number of instructions required to implement the component. To ensure that this is done in a manner that facilitates subsequent steps, the following conventions are used.

### Use instructions consistent with the least complex DSP

In general there’s a wide variation in the capabilities of DSPs, but all DSPs will have to implement the same set of operations to implement the same process whether it’s done in a single instruction or 10. To appropriately model this, all pseudo-code instructions should be written in a manner consistent with the least complex DSP. In theory, this could vary from instruction to instruction, but using instructions consistent with the ARM9 instruction set will generally suffice.

Key examples to be aware of include the way conditional branches are taken (e.g., some require a flag be set, some can be done by directly examining the content of a register) as well as explicit instruction combinations (e.g., a MAC).

### Label instruction lines

In general, the basic equation is modified by eliminating instructions that on a processor will either be unnecessary or combined with another instruction. However, it will sometimes be the case that multiple modifiers for a processor will eliminate the same line. To identify these situations in the equation formulation step, we will associate the eliminated instruction lines with the modifier so that when a duplication occurs, it can be handled as a “synergistic” modifier. Also a different appellation should be adopted for instructions in each loop. Examples used in the following include labeling lines as 1,2,3,… for instructions that occur outside a loop, L1, L2, L3, … for instructions inside a loop, OL1, OL2,… for instructions in an outer-loop and so on.

### Clearly identify assumptions

It is sometimes hard to implement a perfectly generalizable component so some assumptions about a processor’s capabilities must be made (e.g., floating point or 32-bit). When this occurs, make certain that this is clearly identified so it can be built into the component file and thereby excluded from implementation on processors that do not support those assumptions.

### Register conventions

While some DSPS (e.g., C54) can directly manipulate elements in its caches in its instructions, this is the exception. Thus before performing an operation on an element in memory, it should be moved to the local register file (e.g., R1-R16). For processors without register files, these extra cycles will be handled via a modifier that eliminates memory accesses.

Many DSPs assign special meanings to specific registers in their register files (e.g., to store the branch back address, input data pointers, output data pointers, for circular buffering). The specific registers should be ignored in this pseudo-code because of the significant variation from DSP to DSP. Also the pseudo-code should ignore limitations on the number of available registers. In practice this would require additional cycles to interface with cache, but this will be both component and DSP-specific. If need be, a specialized component could be written to address a register-constrained condition.

However, code should account for some instruction(s) to put data into an appropriate register when entering or leaving the procedure. Also when calling another procedure, there should also be an instruction to place the address of where the procedure should return to into the appropriate register or stack.

### Reset internal settings before exiting

It is a good coding practice that whenever a procedure changes an internal setting, this setting should be reset before exiting the procedure. So, for example, if a circular addressing mode is needed in a particular component, the existing mode should be saved upon entry to the procedure and restored upon exit from the procedure.

### Group operations that combine together

To simplify the generation of modifier equations, instructions which group together in known modifiers should be placed sequentially in the pseudo-code. If they cannot be placed sequentially, that is a good indication that the modifier would not apply in that situation.

### Attempt to parameterize as much as possible

One of the goals of this tool is to allow for component file reuse as much as possible. For example, there should be no need to redo the analysis when moving from a filter of length 31 to a filter of length 63. Thus when possible, operations which depend on typical parameterizations of the component should be identified and expressed in terms of that parameter.

Even when the original coder is unaware of a parameterization, loop counters will generally serve as a parameterizable value (e.g., filter length). Note that a loop counter is not necessary for an equivalent parameterization. In fact, on processors where loop control is a significant burden, it is a common practice to unroll loops. In such a case, the pseudo-code might include a comment that the following should be repeated r times or some such. Again to help identify exactly what will be repeated (or looped) it is helpful to use meaningful labels.

### Comment

Reading assembly code is hard (though not as hard as reading machine code!). To promote the sharing of component files and documentation and to make it possible for the original coder to perform validation weeks, if not merely hours, after writing the pseudo-assembly, the pseudo-code should be commented as much as possible. This also has the additional benefit that if a DSP is added to the suite later that has new capabilities, there may be sufficient context to evaluate where it could be applied to previously generated component files.

Note that lines used for commenting should not be added to the instruction / cycle count.

## Creating equations

The primary goal of performing the component analysis is to generate the set of equations that define the component file. The basic operations equation is defined by counting up the number of instructions required to implement the component, parameterized as appropriate. This count should then be subdivided into memory operations, multiplication operations, and other (ALU) operations. This is needed for the VLIW algorithms to run correctly.

Modifier equations should be generated by reviewing the modifiers listed in Section 1.4 and identifying when the requisite conditions are satisfied in the pseudo-code. When this occurs, this should be noted along with the number of instructions that would be eliminated by the presence of that modifier. Each of these modifier equations should be associated with the specific instruction lines that would be eliminated.

Synergistic equations are created by reviewing the list of modifier equations and identifying where modifiers target the same instruction. The synergistic equation should undue the effects of instructions that were double-counted (or triple-counted) by the modifiers.

## Modifiers

Table 1 reproduces the table of modifiers identified in the document entitled “DSP Mappings (Task 1.3a) for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.” In general when pseudo-code is identified as exhibiting the operations listed in the middle column, the effect in the right column is applied to generate a modifier equation. Note that in practice many of these instruction modifiers actually capture multiple instructions (e.g., ABSALU is associated with evaluating the absolute value of the result of any ALU operation, and a MAC can be done with either a multiplication and accumulation or a multiplication and negative accumulation) and that different processors will use different terminology for the same effect (e.g., block repeat versus a zero-overhead loop).

Table 1: Instruction / Cycle modifiers identified from DSP analysis

| Instruction Modifier | Operation | Modeled Effect |
| --- | --- | --- |
| ABSALU | A typical ALU instruction (ADD, SUB) is combined with an absolute value operation. Useful for distance calculations and L1 norms. | Cycles associated with subsequent abs eliminated |
| ADDSUB | The ALU can simultaneously add and subtract two numbers and/or add/subtract two numbers of the native precision. Note, this must be the *same* two numbers, e.g., A+B, A-B | Consecutive adds/subtracts of the same registers eliminated |
| AVG | The processor implements (A+B)/2 | Eliminates a left shift when following an add |
| BDEC | The processor decrements a counter and branches if the counter is non zero | Loop control cycles |
| BPOS | The processor branches on the conditions of registers rather than requiring an instruction to set a flag. | Cycle eliminated for generating branch condition |
| BITR | The processor reverses the bits of a register in a single cycle. Frequently implemented as bit reverse incrementing/arithmetic. | Cycles for process to bit-reverse an indexed array are eliminated. 1 cycle per array element added back in. |
| COND_EXEC | All instructions are executed conditionally | Cycles consumed for short control branches are eliminated |
| COND_MOV | All memory/move operations can be executed conditionally | Cycles consumed for short control branches related to memory are eliminated |
| CPX_MPY | A single cycle complex multiplication. Note that several DSPs have an instruction which implements a complex multiplication (or LMS update or an FIR cycle) but these are multi-cycle instructions. Here, we’re only modeling situations where x_r * y_r – x_i*y_i, x_i*y_r + x_r*y_i. So far, this has also included xyH. | Complex multiplication cycles (6) reduced to 1 per complex multiplication. |
| EXTRACT | The processor is capable of detailed bit manipulation in a single cycle. This takes various forms in different instructions, but a minimum requirement is the ability to extract out a specified set of bits from a word and then pack them into bytes | Bit manipulation cycles cut in half. |
| GMPY | The processor supports Galois Field arithmetic (useful in some error correction codes). | Cycles required to mimic Galois Field arithmetic are eliminated with one cycle per Galois Field arithmetic operation added in. |
| INDEX | Of form *R4[R6]++ | Cycles used to offset a register for a memory operation are eliminated |
| MAC | A single-instruction (functional unit) multiplication and accumulation. Note that when both a multiplier and an ALU are required to implement this, a processor is not considered to exhibit the MAC modifier unless it does not support VLIW | Accumulate cycles eliminated |
| MAX | The processor does the following (and the reverse for a MIN): if A>B, A-> dst | Cycles required to perform move following comparison operation eliminated |
| MAX2 | The process performs the MAX operation for two pairs of words of native precision. | Cycles required to perform both moves and one comparison eliminated |
| MEM2 | The data bus width of a processor is such that a single instruction fetches 2 words. | Memory cycles cut in half (rounded up). |
| MEM4 | The data bus width of a processor is such that a single instruction fetches 4 words. | Memory cycles cut in fourth (rounded up). |
| NOREG | The processor memory maps all registers so there’s no need for an instruction to load registers from memory. Processors which do this, however, tend to clock much slower | All cycles used to move |
| SAD | Sum of absolute differences. | Absolute values removed. A special case of ABSALU. |
| SIDE_SUM | Adds up bytes in a word. | Eliminate cycles of sum of byte-packed words |
| VSL | A process by which a register is shifted left and an input 1 or 0 is appended to the right most bit. Useful for keeping track of paths (saves an instruction) and some bit manipulation operations. | Cycle saved per path update in |
| ZOL | The processor supports a form of zero-overhead (hardware) looping wherein loop instructions are placed in a hardware buffer and repeated a specified number of times. | All loop control cycles eliminated (branch, compare, decrement). One cycle added to set loop counter. |

# Real Filter

This component represents the implementation of a real filter without assumptions about the number or symmetry of the taps and computes a single output for a set of inputs. Note that different structures (e.g., block FIR, symmetric FIRs) will be generally coded in a different manner. Because the process will vary from implementation to implementation, all scaling is assumed to occur outside of this function.

## Pseudocode

Requirements: circular

Parameters: N (length)

y=fir(coef, data, length, offset)

//Set circular buffer params

1	(instruction to store previous setting in local register)

2	(instruction to store buff length)

3	(instruction to turn on circ buff)

4	(instruction to set buffer length)

//Move input parameters to local registers

5	R1 = coef (address)

6	R2 = data (address)

7	R2 = data + offset // needed for circular buffering

8	R3 = length (actual #)

//zero accumulator (typically done by subtracting a register from itself)

9	acc = 0

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label)	R4 = *R1++ // I don’t know of a DSP that doesn’t support postfix addressing,    // but if there is one, a cycle would need to be added here

L2	R5 = *R2++ (

L3	R6 = R5 * R4

L4	acc = acc + R6

L5	R3 = R3 – 1

L6	flag = cmp(R3,0)

L7	if flag (R3==0), branch to loop

//	Move result to output register

10	R_out = acc

//	Restore stuff

11	(instruction to turn reset addressing mode)

12	(instruction to reset buffer length)

13	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 13+ 7 * N |
| Memory | 2*N |
| Multiplication | N |
| ALU | 4*N + 13 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L5, L6 eliminated | (ALU) -2*N |
| BPOS | L6 eliminated | (ALU) – N |
| ZOL | 1 cycle to set register, L5, L6, L7 eliminated | (ALU) 1 – 3*N |
| MAC | L4 eliminated | (ALU) -N |
| MEM2 | MEM cut in half | (MEM) –(2*N)/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(2*N)/4 |
| NOREG | MEM eliminated | (MEM) –(2*N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L5,L6 added back in | (ALU) target+2*N |
| BPOS, BDEC | L6 added back in | (ALU) target+N |
| BPOS, ZOL | L6 added back in | (ALU) target+N |
| BPOS, BDEC, ZOL | L6 eliminated (again) | (ALU) target – N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (2*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(2*N)/4 |

# Complex Filter

This component represents the implementation of a complex filter without assumptions about the number or symmetry of the taps and computes a single output for a set of inputs. Note that different structures (e.g., block FIR, symmetric FIRs) will be generally coded in a different manner. Because the process will vary from implementation to implementation, all scaling is assumed to occur outside of this function.

## Pseudocode

Requirements: circular

Parameters: N (length)

y=fir(coef_real, coef_imag, data_real, data_imag, length, offset)

//Set circular buffer params

1	(instruction to store previous setting in local register) (real)

2	(instruction to store previous setting in local register) (imag)

3	(instruction to store previous setting in local register) (real)

4	(instruction to store previous setting in local register) (imag)

5	(instruction to turn on circ buff) (real)

6	(instruction to turn on circ buff) (imag)

7	(instruction to set buffer length) (real)

8	(instruction to set buffer length) (imag)

//Move input parameters to local registers

9	R1 = coef_real (address)

10	R2 = coef_imag (address)

11	R3 = data_real (address)

12	R4 = data_imag (address)

13	R3 = data_real + offset // needed for circular buffering

14	R4 = data_real + offset // needed for circular buffering

15	R5 = length (actual #)

//zero accumulators (typically done by subtracting a register from itself)

16	acc_real = 0

17	acc_imag= 0

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label)	R6 = *R1++

L2	R7 = *R2++

L3	R8 = *R3++

L4	R9 = *R4++

//end memory fetches

L5	R10 = R1 * R3

L6	R11 = R2 * R4

L7	R12 = R2 * R3

L8	R13 = R1 * R4

L9	acc_real = acc_real + R10

L10	acc_real = acc_real – R11

L11	acc_imag = acc_imag + R12

L12	acc_imag = acc_imag + R13

//	loop control

L13	R5 = R5 – 1

L14	flag = cmp(R5,0)

L15	if flag (R5!=0), branch to loop

//	Move result to output register

18	store real

19	store imag

//	Restore stuff

20	(instruction to turn reset addressing mode) (real)

21	(instruction to turn reset addressing mode) (imag)

22	(instruction to reset buff) (real)

23	(instruction to reset buff) (imag)

24	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 24+ 15 * N |
| Memory | 4*N |
| Multiplication | 4*N |
| ALU | 7*N + 24 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L13-14 eliminated | (ALU) -2*N |
| BPOS | L14 eliminated | (ALU) – N |
| ZOL | 1 cycle to set register, L13-15 eliminated | (ALU) 1 – 3*N |
| MAC | L9-12 eliminated | (ALU) -4*N |
| MEM2 | MEM cut in half | (MEM) –(4*N)/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(4*N)/4 |
| NOREG | MEM eliminated | (MEM) –(4*N) |
| CMPX_MPY | L5-L12 eliminated | (MULT) -3*N, (ALU)-4*N |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L13,L14 added back in | (ALU) target+2*N |
| BPOS, BDEC | L14 added back in | (ALU) target+N |
| BPOS, ZOL | L14 added back in | (ALU) target+N |
| BPOS, BDEC, ZOL | L14 eliminated (again) | (ALU) target – N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (4*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(4*N)/4 |
| CMPX_MPY, MAC | L9-12 added back in | (ALU) target+ 4*N |

# Fast Fourier Transform (FFT)

This component represents the implementation of a computation-in-place radix-2 complex FFT (decimation in time) where all twiddle factors have been precomputed and stored. Note that relevant counters are assumed hard-coded. The first stage of the FFT accesses the input data in bit-reversed fashion, subsequent stages in normal fashion. In practice this means the first pass should be outside of the main loop. For space, this is not actually coded though it is reflected in the equations. Also note that slightly high SNR could be achieved if the FFT input and the output for each stage is scaled by a factor of ½ as opposed to the implicit pre-stage scaling used here. However, this will add 4*log2(N) cycles (right shift and store rather than store).

## Pseudocode

Requirements: bit-reverse addressing, circular addressing

Parameters: N (length) (everything dependent on this is assumed hard coded in)

y=fir(real_data, imag_data, twid_real, twid_imag)

1	R1 = real_data (address)

2	R2 = imag_data (address)

3	R3 = twid_real (address)

4	R4 = twid_imag (address)

//first pass through the input data should be accessed in bit reverse fashion

//after that, should be in normal linear fashion

//that is reflected here for R1, R2

5	(instruction to store previous setting in local register)

6	(instruction to store previous setting in local register)

7	(instruction to turn on bit-reverse add)

8	(instruction to turn on bit-reverse add)

9	(instruction to turn reset addressing mode)

10	(instruction to turn reset addressing mode)

//also there are 3 less instructions used (one less pass through outerloop loop control)

//note that will have a negative effect on modifier equations

11	num_stages = log_2 (N) // outer loop counter (stage), hardcoded

12	data_step = 1

13 	num_DFT = N/2 //hard coded

14	offset = 1 //used for many things

// defines # butterflies in DFT

// also 2*offset*DFT_counter is start address for DFT

// e.g., in stage 0, DFT2, start address is 2*1*2 = 4 (bit reversed since stage 0)

//

//**********************

//OUTER LOOP (STAGE LOOP)

OL1	DFT_count = num_DFT //set up middle loop counter

//**********************

//MIDDLE (DFT) LOOP

//Point to twiddle to W^0

ML1	twid_real_reg = R3 // wrap around

ML2	twid_imag_reg = R4 // wrap around

//calculate initial index for butterfly (k*offset)

ML3	temp1 = Num_DFT - DFT_count //(k=0,1,2,…)

ML4	temp = temp1 * offset

ML5	temp = temp << 1

//initial a address

ML5	a_real_reg = temp

ML6	a_imag_reg = temp

//initial b address

ML7	b_real_reg = a_real_reg + offset

ML8	b_imag_reg = a_imag_reg + offset

ML9	butterfly_count = offset //inner loop counter

//**********************

//INNER LOOP

label: INNER LOOP

//START BUTTERFLY

//A FFT butterfly is implemented as

// A = a + twid*b (complex multiplication)

// B = a – twid*n (complex multiplication)

//butterfly (there are some chips where the following is collapsed down to 2-4 cycles, though not included //in this survey. Labeled separately to make it easier to adjust later

B1	twid_real_val = *twid_real_reg

B2	twid_imag_val = *twid_imag_reg

B3	a_real = *a_real_reg

B4	a_imag = *a_imag_reg

B5	b_real = *b_real_reg

B6	b_imag = *b_imag_reg

//complex multiplication b*twid

B7	R1 = b_real*twid_real_val

B8	R2 = b_imag*twid_imag_val

B9	b_mod_real = R1-R2

B10	R1 = b_real*twid_imag

B11	R2 = b_imag*twid_imag

B12	b_mod_imag = R1 + R2

//A

B13	temp_a_real = a_real + b_mod_real

B14	temp_a_imag = a_imag +b_mod_imag

B15	*a_real_reg++ = temp_a_real

B16	*a_real_imag++ = temp_a_imag

//B

B17	temp_b_real = a_real - b_mod_real

B18	temp_b_imag = a_imag -b_mod_imag

B19	*b_real_reg++ = temp_b_real

B20	*b_real_imag++ = temp_b_imag

//END BUTTERFLY

//**********************

IL1	twid_real_reg = twid_real_reg + num_DFTs // wrap around

IL2	twid_real_imag = twid_real_imag + num_DFTs // wrap around

IL3	butterfly_count = butterfly_count – 1

IL4	flag = cmpeq(butterfly_count,0)

IL5	if flag(butterfly _count !=0), branch INNER LOOP

//END INNER LOOP

//**********************

ML10	DFT_count = DFT_count – 1

ML11	flag = cmpeq(DFT_count,0)

ML12	if flag(DFT _count !=0), branch MIDDLE LOOP

//END MIDDLELOOP

//**********************

//resize for next stage

OL2	num_DFT = num_DFT >> 1; //e.g., 8,4,2,1

OL3	offset = offset << 1;

OL4	stage_size = stage_size << 1;

OL5	stage_counter = stage_counter – 1

OL6	flag = cmpeq(stage_counter,0)

OL7	if flag(stage_counter !=0), branch OUTER LOOP

//END OUTER LOOP

//**********************

//	Restore stuff

16	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 15+ 7*log2(N) + (N-1)* 12+ (20 + 6)*log2(N) |
| Memory | 10*log2(N) |
| Multiplication | (N-1)*1 + 4*log2(N) |
| ALU | 15+7*log2(N) + (N-1)*11+12*log2(N) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL5,6 ML10,11, IL3,4 eliminated | (ALU) -2*(log2(N) + (N-1) + log2(N)) |
| BPOS | OL6 ML11, IL4 eliminated | (ALU) – (log2(N) + (N-1) + log2(N)) |
| ZOL | OL5-7 ML10-12, IL3-5 eliminated | (ALU) (log2(N) + (N-1)) – 3* log2(N) + (N-1) + log2(N) |
| MEM2 | MEM cut in half | (MEM) -(10*log2(N))/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(10*log2(N))/4 |
| NOREG | MEM eliminated | (MEM) -(10*log2(N)) |
| CMPX_MPY | B7-11 eliminated | (MULT) -3* log2(N), (ALU)-4*log2(N) |
| MAC | B9, B12 eliminated | (ALU) -2*log2(N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL5,6 ML10,11, IL3,4 added back in | (ALU) target+2*(log2(N) + (N-1) + log2(N)) |
| BPOS, BDEC | OL6 ML11, IL4 added back in | (ALU) target+(log2(N) + (N-1) + log2(N)) |
| BPOS, ZOL | OL6 ML11, IL4 added back in | (ALU) target+(log2(N) + (N-1) + log2(N)) |
| BPOS, BDEC, ZOL | OL6 ML11, IL4 eliminated (again) | (ALU) target – (log2(N) + (N-1) + log2(N)) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (10*log2(N))/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(10*log2(N))/4 |
| CMPX_MPY, MAC | B9, B12 added back in | (ALU) target-2*log2(N) |

# LMS Equalizer (real)

This component represents the implementation of a real least-mean-squares equalizer which generates outputs on a symbol by symbol basis and updates the coefficients with the error calculated externally. Because the process will vary from implementation to implementation, all scaling is assumed to occur outside of this function. Note that circular buffering is not generally done with LMS equalization because it is run at the symbol rate rather than at the sample rate.

## Pseudocode

Parameters: N (filter length)

y=lms(coef, data, length, error, step)

//Move input parameters to local registers

1	R1 = coef (address)

2	R2 = data (address)

3	R3 = length (actual #)

4 	R4 = error

5	R5 = step

//zero accumulator (typically done by subtracting a register from itself)

6	acc = 0 //an ALU operation

//filtering operation

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

//(loop1 label)

L1,1	R7 = *R1++

L1,2	R8 = *R2++

L1,3	R9 = R7 * R8

L1,4	acc = acc + R9

L1,5	R3 = R3 – 1

L1,6	flag = cmp(R3,0)

L1,7	if flag (R3!=0), branch to loop1

//adjust coefficients

7	R6 = step * error // no need to calculate this everytime

8	R6 = R6 * acc //(y*err * weight)

9	R1 = coef (address)//reset pointers

10	R2 = data (address)

11	R3 = length

//Loop through and update coefficients

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

//loop2

L2,1	R7 = *R1 // note no postfix adjustment yet

L2,2	R8 = *R2++ //x[k]

L2,3	R9 = R8 * R6 //x [k] * y* err * weight

L2,4	R7 = R9 + R7 //h[k] + x [k] * y* err * weight

L2,5	*R1++ = R7 // h[k] + x [k] * y*err * weight

L2,6	R3 = R3 – 1

L2,7	flag = cmp(R3,0)

L2,8	if flag (R3!=0), branch to loop2

//	Move result to output register

12	R_out = acc

//	Restore stuff

13	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 13+ N * 15 |
| Memory | 5*N |
| Multiplication | 2*N + 2 |
| ALU | 8*N + 11 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L1,5-6; L2,6-7 eliminated | (ALU) -4*N |
| BPOS | L1,6; L2,7 eliminated | (ALU) -2*N |
| ZOL | L1,5-7; L2,6-8 eliminated + setup | (ALU) 2 - 6*N |
| MAC | L1,4; L2,4 eliminated | (ALU) -2*N |
| MEM2 | MEM cut in half | (MEM) -(5*N)/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(5*N)/4 |
| NOREG | MEM eliminated | (MEM) -(5*N ) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L1,5-6; L2,6-7 added back | (ALU) target+6*N |
| BPOS, BDEC | L1,6; L2,7 eliminated | (ALU) target+2*N |
| BPOS, ZOL | L1,6; L2,7 eliminated | (ALU) target+2*N |
| BPOS, BDEC, ZOL | L1,6; L2,7 eliminated (again) | (ALU) target – 2*N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (5*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(5*N)/4 |

# Taylor Series

This component represents the implementation of a Taylor series expansion of cos(x). It assumes reciprocals of the factorials are precomputed and stored (with appropriate signs). All scaling is expected to occur outside of this component. Note the # terms refers to the number of terms which should be used in the calculation which means the least term will be approximzately 2*term.

## Pseudocode

Parameters: N (terms)

y=cos_tayl(x, rcp, length)

//Move input parameters to local registers

1	R1 = x (value)

2	R2 = rcp (address)

3	R3 = length (actual #)

//zero accumulator (typically done by subtracting a register from itself)

4	acc = 1 //scaled as need be

//calculate output

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label)	R7 = *R2++ //load rcp

L2	R1 = R1*R1 //even exponents

L3	R9 = R1 * R7

L4	R1 = R1*R1 //odd exponent

L5	acc = acc + R9

L6	R3 = R3 – 1

L7	flag = cmp(R3,0)

L8	if flag (R3==0), branch to loop

//	Move result to output register

5	R_out = acc

6	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 6 + N*8 |
| Memory | N |
| Multiplication | 3*N |
| ALU | 4*N + 6 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L,6-7 eliminated | (ALU) -2*N |
| BPOS | L7 eliminated | (ALU) – N |
| ZOL | L6-8 eliminated + setup | (ALU) 1 – 3*N |
| MAC | L5 eliminated | (ALU) -N |
| MEM2 | MEM cut in half | (MEM) -(N)/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(N)/4 |
| NOREG | MEM eliminated | (MEM) -(N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L6-7 added back | (ALU) target+2*N |
| BPOS, BDEC | L7 added back | (ALU) target+N |
| BPOS, ZOL | L7 added back | (ALU) target+N |
| BPOS, BDEC, ZOL | L7 eliminated (again) | (ALU) target – N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N)/4 |

# CORDIC

This component represents the implementation of a CORDIC algorithm operating in normal mode (as opposed to hyperbolic or linear modes) where arctangent values have been precomputed and stored. Note K is given by which is assumed to have been precalculated external to this function (resolution is not frequently changed from call to call).

## Pseudocode

Parameters: N (length, i.e., number of iterations)

result=CORDIC(theta, K, length, atan)

//Move input parameters to local registers

1	z = theta (value)

2	R2 = K

3	R3 = length (actual #)

4	R4 = atan (address)

//assign initial values (registers)

4	x = K

5	y = 0

6	iter = 0

//calculate output

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label)  R5 = RSH (y,iter)

L2	R6 = RSH (x,iter)

L3	R7 = *R4++

L4	flag = cmp(R3,0)

L5	if flag (R3<0), branch to label 2

L5a	x = x – R6

L6a	y = y + R5

L7a	z = z – R7

L7.5a	Branch to label 3

(label2)

L5b	x = x + R6

L6b	y = y – R5

L7b	z = z + R7

(label 3)

L8	R3 = R3 – 1

L9	flag = cmp(R3,0)

L10	if flag (R3!=0), branch to loop

//	Move result to output register

7	store x (cos(theta))

8	store y (sin(theta))

9	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 9 + N*10.5 (L7.5a only executed half of times) |
| Memory | N |
| Multiplication | 0 |
| ALU | 9.5*N + 9 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L8,9 eliminated | (ALU) – 2*N |
| BPOS | L4, L9 eliminated | (ALU) – 2*N |
| ZOL | L8-10 eliminated + setup | (ALU) 1 – 3*N |
| COND_EXEC | L7.5 eliminated | (ALU) -0.5*N |
| MEM2 | MEM cut in half | (MEM) –(N)/2 |
| MEM4 | MEM cut in fourth | (MEM) –3*(N)/4 |
| NOREG | MEM eliminated | (MEM) –(N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L8-9 added back | (ALU) target+2*N |
| BPOS, BDEC | L9 added back | (ALU) target+N |
| BPOS, ZOL | L9 added back | (ALU) target+N |
| BPOS, BDEC, ZOL | L9 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N)/4 |

# Interleaver

This component represents the implementation of a block interleaver which is given an input linear array of data, an output linear array of data and a mapping array.

## Pseudocode

Parameters: N (length, i.e., number of iterations)

interleaver(x, y, length, map)

//Move input parameters to local registers

1	R1 = x (address)

2	R2 = y (address)

3	R3 = length (actual #)

4	R4 = map (address)

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label) R5 = *R1++ (load x)

L2	R6 = *R4++ (load map – address offset)

L3	R7 = R2 + R6 (offset the address)

L4	*R7 = R5 (store x[k] in y[map])

L5	R3 = R3 – 1

L6	flag = cmp(R3,0)

L7	if flag (R3==0), branch to loop

5	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 5 + N*7 |
| Memory | 3*N |
| Multiplication | 0 |
| ALU | 4*N+5 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L5,6 eliminated | (ALU) -2*N |
| BPOS | L6 eliminated | (ALU) – N |
| ZOL | L5-6 eliminated + setup | (ALU) 1 – 3*N |
| INDEX | L3 eliminated | (MEM) -N |
| MEM2 | MEM cut in half | (MEM) -(3*N)/2 |
| MEM4 | MEM cut in fourth | (MEM) -3*(3*N)/4 |
| NOREG | 1-4; L1, L4 eliminated | (MEM) -3*N - 4 |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L5-6 added back | (ALU) target+2*N |
| BPOS, BDEC | L6 added back | (ALU) target+N |
| BPOS, ZOL | L6 added back | (ALU) target+N |
| BPOS, BDEC, ZOL | L6 eliminated (again) | (ALU) target – N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(3*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(3*N)/4 |

# DeInterleaver

This component represents the implementation of a block deinterleaver which is given an input linear array of data, an output linear array of data and a mapping array. Uses the same map as for the interleaver, i.e., moves y into x.

## Pseudocode

Parameters: N (length, i.e., number of iterations)

deinterleaver(x, y, length, map)

//Move input parameters to local registers

1	R1 = x (address)

2	R2 = y (address)

3	R3 = length (actual #)

4	R4 = map (address)

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

L1	(loop label) R6 = *R4++ (load map – address offset)

L2	R7 = R2 + R6 (offset the address)

L4	R5 = *R7 (fetch y[map])

L4	(loop label) *R1++ = R5 (store in x[k])

L5	(label 3) R3 = R3 – 1

L6	flag = cmp(R3,0)

L7	if flag (R3==0), branch to loop

5	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 5 + N*7 |
| Memory | 3*N |
| Multiplication | 0 |
| ALU | 4*N+5 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L5,6 eliminated | (ALU) -2*N |
| BPOS | L6 eliminated | (ALU) – N |
| ZOL | L5-6 eliminated + setup | (ALU) 1 – 3*N |
| INDEX | L3 eliminated | (MEM) -N |
| MEM2 | MEM cut in half | (MEM) -(3*N)/2 |
| MEM4 | MEM cut in fourth | (MEM) –3*(3*N)/4 |
| NOREG | 1-4; L1, L4 eliminated | (MEM) –3*N |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L5-6 added back | (ALU) target+2*N |
| BPOS, BDEC | L6 added back | (ALU) target+N |
| BPOS, ZOL | L6 added back | (ALU) target+N |
| BPOS, BDEC, ZOL | L6 eliminated (again) | (ALU) target – N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(3*N )/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(3*N)/4 |

# CIC Filter (Interpolator, M=1)

This component represents the implementation of a block CIC interpolator with differential delay of 1 (M=1 in traditional notation). Note that doing more than M=1 will generally significantly increase the cycle count (gotta calculate address offsets), but that increasing M beyond 1 will not increase cycles (though it does increase memory requirements).

Note: most CIC implementations have their stage loops unrolled (typically length <5 well within typically available # registers) and this is reflected in the implementation. Also note that while we’re still assuming that scaling occurs outside of the CIC filter (this can be quite large for CIC decimators).

## Pseudocode

Parameters: N (block length of input data)

S (number of stages)

R (upconversion rate)

cic_interpolator(x, y, length, I_reg, C_reg, R)

//Move input parameters to local registers

1	R1 = x (address)

2	R2 = y (address)

3	R3 = length (actual #)

4	R4 = I_reg (address)

5	R5 = C_reg(address)

6	R6 = R (actual #)

//Note inherent assumption that length > 0

//Note for loops are implemented as conditional branches in assembly

OL1	(loop label) R7 = *R1++ (load x)

//Comb Stage (showing 3 unrolled could be S)

OL2,1	R8 = *R4++ (get stored value stage 1)

OL2,2	R9 = *R4++ (get stored value stage 2)

OL2,3 	R10 = *R4++ (get stored value stage 3)

OL3,1	R10 = R9 - R10 (evaluate comb stage 3)

OL3,2	R9 = R8 - R9 (evaluate comb stage 2)

OL3,3	R8 = R7 – R8 (evaluate comb stage 1)

OL4,1	*R4-- = R10 (store comb stage 1)

OL4,2	*R4-- = R9 (store comb stage 2)

OL4,3	*R4-- = R8 (store comb stage 3)

//Integrator Stage (showing 3 unrolled could be S)

//There’s some accumulated zeros from zero-stuffing that can be saved in the first integrator with a less

// general implementation

OL5	R7 = R6

(loop2)

IL1,1	R8 = *R5++ (get stored value stage 1)

IL1,2	R9 = *R5++ (get stored value stage 2)

IL1,3 	R11 = *R5++ (get stored value stage 3)

IL2,1	R11 = R11 + R9 (evaluate comb stage 3)

IL2,2	R9 = R8 + R9 (evaluate comb stage 2)

IL2,3	R8 = R10 + R8 (evaluate comb stage 1)

IL3,1	*R5-- = R11 (store comb stage 3)

IL3,2	*R5-- = R9 (store comb stage 2)

IL3,3	*R5-- = R8 (store comb stage 1)

IL4	*R2+= = R11 (store output)

IL5	R7 = R7 – 1

IL6	flag = cmp(R7,0)

IL7	if flag (R7==0), branch to loop2

//outer loop control

OL6	(label 3) R3 = R3 – 1

OL7	flag = cmp(R3,0)

OL8	if flag (R3==0), branch to loop

7	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 7+ N*(3*S +5) + N*R*(3*S +4) |
| Memory | N*(2*S+1) + N*R(2*S+1) |
| Multiplication | 0 |
| ALU | 7+N*(S+4) + N*R*(S+3) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | IL5,6, OL6,7 eliminated | (ALU) -2*N*(1+R) |
| BPOS | IL6, OL7 eliminated | (ALU) -N*(1+R) |
| ZOL | IL5-7, OL6 eliminated + setup | (ALU) 1 -3*N*R |
| MEM2 | MEM cut in half | (MEM) –N*S*(1+R) -N/2*(1+R) |
| MEM4 | MEM cut in fourth | (MEM) – 3*(N*(2*S+1) + N*R(2*S+1))/4 |
| NOREG | MEM eliminated | (MEM) -(N*(2*S+1) + N*R(2*S+1)) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | IL5,6, OL6,7 added back | (ALU) target+2*N*(1+R) |
| BPOS, BDEC | IL6, OL7 added back | (ALU) target+N*(1+R) |
| BPOS, ZOL | IL6, OL7 added back | (ALU) target+N*(1+R) |
| BPOS, BDEC, ZOL | IL6, OL7 eliminated (again) | (ALU) target - N*(1+R) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +N*S*(1+R) +N/2*(1+R) |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N*(2*S+1) + N*R(2*S+1))/4 |

# CRC Encoder

This component represents the implementation of a CRC encoder of a message with length M bits with an encoding polynomial of order r where r<=16. The implementation assumes a register width of 16, While this is a straight mapping from a typical hardware implementation (bit-by-bit), a more efficient CRC encoder can be implemented using LUTs when the CRC has a relatively low order (e.g.,  <=5). However, the LUT approach is impractical for high order encoders (e.g., a CRC with  order 16 would need 2^(16+) entries. Note for very short messages, it would more efficient to specify # of bits instead of # of 16-bit words. Also note that if program memory were more important than cycles, this could be rewritten to loop instead of unrolling the loop. Also note that the input message should be padded with zeros at the end to effect outputting of the CRC check bits. Finally, the polynomial should include the leading 1

## Pseudocode

Parameters: M (length of message)

remainder = crc_encoder(message, p, length, output,)

//Move input parameters to local registers

1	R1 = message

2	R2 = p (encoding polynomial)

3	R3 = length (#16 bit words)

4	R10 = output

5	R6 = 0 //relax CRC register

6	R7 = 0 //output register

loop1:

L1	R5 = *R1++ //load first 16-bit word

//unrolled 16 times

L1R	R9 = R5&(2^16-1) //left most

L2R	flag = cmpgt(R9,0)

L3R	if flag(R9==0) branch label1

L3.5R	R6 = R6 XOR R2

Label 1:

L4R	R7 <<1

L5R	R9 = R6&1

L6R	R7 = R7 XOR R9

L7R	R5 = R5 << 1

L8R	R6 = R6 >> 1 //update shift register

L2	*R10++ = R7 //output word

//Note inherent assumption that length > 2

L3	R3 = R3 – 1

L4	flag = cmp(R3,0)

L5	if flag (R3==0), branch to loop

7	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 7 +8.5*M*16 + M*5 |
| Memory | 2*M |
| Multiplication | 0 |
| ALU | 7+8.5*M*16 + 3*M |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L3,4 eliminated | (ALU) -2*M |
| BPOS | L4, L2R eliminated | (ALU) -M-16*M |
| ZOL | L2-4 eliminated + setup | (ALU) 1 -3*M |
| COND_EXEC | L3.5R eliminated | (ALU) -8*M |
| EXTRACT | L5R, L7R eliminated | (ALU) -2*16*M |
| MEM2 | MEM cut in half | (MEM) – (2*M)/2 |
| MEM4 | MEM cut in fourth | (MEM) – 3*(2*M)/4 |
| NOREG | MEM eliminated | (MEM) -(2*M) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L2,3 added back | (ALU) target+2*M |
| BPOS, BDEC | L3 added back | (ALU) target+M |
| BPOS, ZOL | L3 added back | (ALU) target+M |
| BPOS, BDEC, ZOL | L3 eliminated (again) | (ALU) target - M |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(2*M)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(2*M)/4 |

# CRC Decoder

This component represents the implementation of a CRC decoder of a message with length M bits with an encoding polynomial of order r where r<=16. Note that a CRC decoder is a CRC encoder, except that the input includes check bits (implicit to the output of the encoder above).

## Pseudocode

Parameters: M (length of message)

remainder = crc_encoder(message, p, length, output,)

//Move input parameters to local registers

1	R1 = message

2	R2 = p (encoding polynomial)

3	R3 = length (#16 bit words)

4	R10 = output

5	R6 = 0 //relax CRC register

6	R7 = 0 //output register

loop1:

L1	R5 = *R1++ //load first 16-bit word

//unrolled 16 times

L1R	R9 = R5&(2^16-1) //left most

L2R	flag = cmpgt(R9,0)

L3R	if flag(R9==0) branch label1

L3.5R	R6 = R6 XOR R2

Label 1:

L4R	R7 <<1

L5R	R9 = R6&1

L6R	R7 = R7 XOR R9

L7R	R5 = R5 << 1

L8R	R6 = R6 >> 1 //update shift register

L2	*R10++ = R7 //output word

//Note inherent assumption that length > 2

L3	R3 = R3 – 1

L4	flag = cmp(R3,0)

L5	if flag (R3==0), branch to loop

7	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 7 +8.5*M*16 + M*5 |
| Memory | 2*M |
| Multiplication | 0 |
| ALU | 7+8.5*M*16 + 3*M |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L3,4 eliminated | (ALU) -2*M |
| BPOS | L4, L2R eliminated | (ALU) -M-16*M |
| ZOL | L2-4 eliminated + setup | (ALU) 1 -3*M |
| COND_EXEC | L3.5R eliminated | (ALU) -8*M |
| EXTRACT | L5R, L7R eliminated | (ALU) -2*16*M |
| MEM2 | MEM cut in half | (MEM) – (2*M)/2 |
| MEM4 | MEM cut in fourth | (MEM) – 3*(2*M)/4 |
| NOREG | MEM eliminated | (MEM) -(2*M) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L2,3 added back | (ALU) target+2*M |
| BPOS, BDEC | L3 added back | (ALU) target+M |
| BPOS, ZOL | L3 added back | (ALU) target+M |
| BPOS, BDEC, ZOL | L3 eliminated (again) | (ALU) target - M |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(2*M)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(2*M)/4 |

# Convolutional Encoder

This component represents the implementation of a convolutional encoder of a message with length M 16-bits words of constraint length K (K<=16) and rate r (of form 1/r – puncturing can occur in a separate process). The encoding is assumed to be hardcoded to minimize cycles. Note: we’re encoding 16 bits per loop.

## Pseudocode

Parameters: 	M (# of 16-bit words in message – round up)

K (constraint length)

taps (total # of XOR taps for encoding polynomials)

r (rate)

crc_encoder(message, length, g0, …, gr)

//Move input parameters to local registers

1	R1 = message (address)

2	R3 = length  (#16 bit words)

//Note inherent assumption that length > 2

3	R4 = 0 //buffer is initially empty

//	set pointers to output registers (r lines)

4	Rx1 = g0

5	Rx2 = g1

L1	loop1: 	R5 = *R1++ // storing word to be shifted in

//repeat K  times (here we’ll assume K = 3)

L21	R6 = R4 >> 1

L22	R2 = R5 & 1 //these three cycles are easily eliminated collapsed to one with good bit control

L23	R2 = R2 << 15

L24	R6 = R6 XOR R2

L31	R7 = R4 >> 2

L32	R2 = R5 & 3 //these three cycles are easily eliminated collapsed to one with good bit control

L33	R2 = R2 << 14

L34	R7 = R7 XOR R2

L41	R8 = R4 >> 3

L42	R2 = R5 & 7 //these three cycles are easily eliminated collapsed to one with good bit control

L43	R2 = R2 << 13

L44	R6 = R6 XOR R2

//repeat r times

//repeat taps -1 times

L51	R9= R8 XOR R6 // g0 = 1+x+x^2

L52	R9 = R9 XOR R5

L53	*g0++ = R9 //store 16-bit result in output word

// note total # operations will be equal to taps

L6	R4 = R5

//loop control

L7	R3= R3 – 1 //decrement count

L8 	flag = Compare R3,0

L9	If flag, branch Loop

//Epilog

6	R5 = 0

(effectively repeat the loop except for L1, L6-L9)

7	(instruction to branch back)

## Raw Operations Equation

| Class | Equation |
| --- | --- |
| Raw | 5 + r + (M-1)*(5+4*K+r*taps) + (4*K+r*taps) |
| Memory | r*(M) + M-1 |
| Multiplication | 0 |
| ALU | 5 + r+ M*(K*4+(taps-1)*r) + (M-1)*4 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L7,L8 eliminated | (ALU) -2*(M – 1) |
| BPOS | L8 eliminated | (ALU) –(M-1) |
| ZOL | L7-9 eliminated + setup | (ALU) -3*(M-1)+1 |
| EXTRACT | L22-24 et al collapsed to single cycle | (ALU) -2*M*K |
| MEM2 | MEM operations cut in half | (MEM) – (r*(M) + M-1)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(r*(M) + M-1)/4 |
| NOREG | MEM operations eliminated | (MEM) – (r*(M) + M-1) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L7,L8 added back | (ALU) target +2*(M – 1) |
| BPOS, BDEC | L8 added back | (ALU) target+(M-1) |
| BPOS, ZOL | L8 added back | (ALU) target+(M-1) |
| BPOS, BDEC, ZOL | L8 eliminated (again) | (ALU) target - (M-1) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (r*(M) + M-1)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(r*(M) + M-1)/4 |

# Viterbi Decoder (rate 1/r, hard decisions, traceback = 32)

This component represents the implementation of a Viterbi decoder with hard decisions for a rate 1/r code without puncturing. The transition matrix (which states go to which states) and the output word matrix (what words would be output by those transitions) should be predefined and passed in and all memory allocations performed externally. For simplicity, the input receive vector is assumed to be grouped into words = of r bits  without packing. Note that traceback is dramatically simplified by assuming that the traceback length is less than the register width. Finally note that this should not be used with codes with K>5 (there’s a rule of thumb for traceback that path lengths > 5.8xK for good SNR)

## Pseudocode

Parameters: num_states

N (length > 32)

r

A Viterbi decoder is a very involved piece of software. For any hope of readability, the following conventions were changed to improve readability. First, variable names instead of generic register names were used. Second, portions of the code were separated out into distinct subsections. None of these subsection are intended to be treated as independent functions/procedures and are instead intended to be substituted in where indicated.

=======================

viterbi_decoder(rcv_vect, length, output_vect, transition_matrix, input_output_matrix)

//initialize metrics and paths

VD1	v = base_address

VD2	*v++ =  0 //0 node is initial node

VD3	reg_bank = base_address

VD4	*reg_bank++ = 0 // initial bit is 0

VD5	rcv_reg = rcv_vect

//VD4	output_reg = output_vect // assigned later when needed

VD6	transition_reg = transition_matrix

VD7	output_matrix = input_output_matrix

//***************

// INITIALIZATION LOOP

VD8	tmp = num_states-1

LABEL: INIT_LOOP

INL1	*v++ = MAX_NEG //hardcoded to largest negative

INL2	*reg_bank++ = 0

INL3	tmp = tmp-1

INL4	flag = cmpeq(tmp,0)

INL5	if !flag, branch init_loop

// END INITIALIZATION LOOP

//************

//****************

// MAIN LOOP

VD9	main_ctr = length

LABEL: MAIN_LOOP

MNL1	v = 0 address //repoint to beginning

MNL2	reg_bank = 0 address // repoint to beginning

(Branch Metric Unit) //advances rcv_sample pointer, assigns values to output_metric array

(Path Metric Unit)

//COPY LOOP

//Copy results from Path Metric Unit

MNL3	state_cnt = num_states

LABEL:	COPY_LOOP

CPY L1	tmp_V = *temp_V++

CPY L2	tmp_reg = *temp_reg_bank++

CPY L3	*V++ = tmp_V

CPY L4	*reg_bank++ = tmp_reg

CPY L5	state_cnt = state_cnt – 1

CPY L6	flag = cmpeq(state_cnt,0)

CPY L7	if !flag, branch copyr loop

//END COPY LOOP

MNL4	Flag = cmplt(main_ctr,31) // main_ctr > 31 (full register) ??

MNL5	If Flag, branch MNL6

(Traceback Unit)

MNL6	main_ctr = main_ctr -1

MNL7	flag = cmpeq(main_ctr,0)

MNL8	if !flag, branch init_loop

// MAIN LOOP

//****************

//****************

// FLUSH REGISTERS

FR1	FR_cnt = 31 //one less than reg width

FR2	temp_reg = reg_bank + ind

FR3	temp_val = *temp_reg

LABEL: FR_LOOP

FRL1	temp_val = temp_val << 1

FRL2	temp_val2 = temp_val

FRL3	temp_val2 = tempval2 >> 31

FRL4	temp = temp_val2 & 1

FRL5	*output_vect++ = temp

FRL6	FR_cnt = FR_cnt - 1

FRL7	flag = cmpeq(FR_cnt, 0)

FRL8	if !flag, branch FR_LOOP

VD10	(Instruction to branch back)

Hard_metric(rcv_samp, ideal)

HM1	Tmp = xor(rcv_word,ideal)

HM2	Acc = 0

//repeat #bits per word (cept once)

W1	Acc = acc+tmp&1

W2	Tmp>>1

===============================

BRANCH METRIC UNIT (effectively)

BMU1		state_cnt = num_states //hard coded

BMU2		Rcv_word = *rcv_samp++

BMU3		output_reg = input_output_matrix

//OUTER LOOP (STATES)

LABEL: OUTER_LOOP

//INNER LOOP (UNROLL twice, no actual loop control)

//2x following

BMU IL1		ideal = *output_reg++

(Hard_metric)

BMU IL2		*output_metric++ = Acc

BMU OL	1	state_cnt = state_cnt – 1

BMU OL2	flag = cmpeq(state_cnt,0)

BMU OL3	if !flag, branch outer loop

====================

Add-Compare-Select

ACS1	Temp0 = V1 + M1 //metric for path with new metric V1 and old M1

ACS2	Temp1 = V2 + M2//metric for path with new metric V2 and old M2

ACS3	Flag = cmp(temp0,temp1)

ACS4	If flag branch ACS8 //can’t quite do this with just a MAX

ACS4.1	Rtn_v = temp0

ACS4.2	Rtn_index = 0

ACS5	branch ACS7

ACS6	Rtn_v = temp1

ACS6.1	Rtn_index = 1

ACS7	//really first thing out of ACS – not counted as an actual instruction

===============================

PATH METRIC UNIT (effectively)

PMU1		PMU_cnt = num_states //hard coded

PMU2		offset = 0

LABEL: PMU_LOOP

//GET INDICES FOR METRICS (WHAT NODE BRANCHES INTO WHAT NODE?)

PMU L1 		ind1 = *temp_transition_matrix++

PMU L2 		ind2 = *temp_transition_matrix--

//GET METRICS FOR LEAD IN NODES

PMU L3		V_temp = V + ind1 //INDEX

PMU L4		V1 = *V_temp

PMU L5		V_temp = V + ind2 //INDEX

PMU L6		V2 = *V_temp

PMU L8		M_temp = output_metrics + ind1

PMU L9		M_temp = output_metrics + offset //INDEX

PMU L10	M1 = *M_temp

PMU L11	M_temp = output_metrics + ind2

PMU L12	M_temp = output_metrics + offset //INDEX

PMU L13	M2 = *M_temp

(ACS CALCULATION)

//PMU L14	*temp_v++ =Rtn_v //actually done in ACS. commented here for clarity

PMU L14	temp_transition_matrix = temp_transition_matrix + Rtn_index

PMU L15	ind2 = *temp_transition_matrix

PMU L16	reg_bank_reg = reg_bank + ind2 // INDEX

PMU L17	reg_bank_val = *reg_bank_reg

PMU L18	reg_bank_val = reg_bank_val << 1 //(zero fill)

PMU L19	reg_bank_val = reg_bank_val OR offset //eliminated with VSL instruction

PMU L20	*temp_reg_bank++ =reg_bank_val

PMU L21	offset = offset XOR 1 // note, this is very much not an add as it’s supposed to toggle the value

PMU L22	PMU_cnt = PMU_cnt - 1

PMU L23	flag = cmpeq(PMU_cnt, 0)

PMU L24	if !flag, branch PMU_LOOP

============================

===============================

TRACEBACK UNIT (effectively)

(FIND_MAX)

TBU1	temp_reg = reg_bank + ind

TBU2	temp_val = *temp_reg

TBU3	temp_val2 = temp_val >> 31 //extract left most bit

TBU4	temp_val = temp_val2 & 1

TBU6	*output_vect++ = temp_val

====================

===================

find_max(int *V_vect)

MAX1	max_cnt = num_states

MAX2	max = 0

MAX3	ind = 0

MAX4	V_vect = V //address

LABEL: MAX_LOOP

MAXL1	tmp = *V_vect++

MAXL2	flag = cmpgt(tmp,max)

MAXL3	if !flag, branch MAXL6

MAXL4	ind = num_states – max_cnt

MAXL5	max = tmp

MAXL6	max_cnt = max_cnt – 1

MAXL7	flag = cmpeq(max_cnt, 0)

MAXL8	if !flag, branch MAX_LOOP

## Raw Operations

To make these estimations readable, the following breaks down the Viterbi mapping operations by module along with the number of times that module is called.

### Find Max

| Class | Equation |
| --- | --- |
| # Called | N-31 |
| Raw | 4 + 8*num_states |
| Memory | num_states |
| Multiplication | 0 |
| ALU | 4 + 7*num_states |

### Traceback Unit

| Class | Equation |
| --- | --- |
| # Called | N - 31 |
| Raw | 6 |
| Memory | 2 |
| Multiplication | 0 |
| ALU | 4 |

### Add Compare Select

| Class | Equation |
| --- | --- |
| # Called | N * num_states |
| Raw | 4+3*0.5+2*0.5 = 6.5 |
| Memory | 0 |
| Multiplication | 0 |
| ALU | 6.5 |

### Path Metric Unit

| Class | Equation |
| --- | --- |
| # Called | N |
| Raw | 2+num_states*24 |
| Memory | num_states*10 |
| Multiplication | 0 |
| ALU | 2+num_stats*14 |

### Hard metric

| Class | Equation |
| --- | --- |
| # Called | N*num_states*2 |
| Raw | 2+rate*2 -1 |
| Memory | 0 |
| Multiplication | 0 |
| ALU | 2+rate*2 -1 |

### Branch Metric Unit

| Class | Equation |
| --- | --- |
| # Called | N |
| Raw | 3+num_states*(4+3) |
| Memory | 1 +num_states*4 |
| Multiplication | 0 |
| ALU | 2+num_states*3 |

### Main Decoder

| Class | Equation |
| --- | --- |
| # Called | 1 |
| Raw | 13 +(num_states-1)*5 + N*(8 + num_states*7) + 3 + 8*31 |
| Memory | 3+ (num_states-1)*2 + N*(2 + num_states*4) + 0 + 1*31 |
| Multiplication | 0 |
| ALU | 10 + (num_states-1)*3 + N*(6 + num_states*3) + 31*7 |

### Integrated Equations

| Class | Equation |
| --- | --- |
| Raw | [13 +(num_states-1)*5 + N*(8 + num_states*7) + 3 + 8*31] + N*[3+num_states*(4+3)] + N*num_states*2*[2+rate*2 -1] + N*[2+num_states*24] + N*num_states*6.5 + 6*(N-31) + (N-31)*(4+8*num_states) |
| Memory | 3+ (num_states-1)*2 + N*(2 + num_states*4) + 1*31 + N*[1 +num_states*4] + N*num_states*2*0 + N * num_states*10 + 0 + (N-31)*2 + (N-31)*num_states |
| Multiplication | 0 |
| ALU | 10 + (num_states-1)*3 + N*(6 + num_states*3) + 31*7 + N*(2+num_states*3) + N*num_states*2*(2+rate*2-1) + N*(2+num_states*14) + N*num_states*6.5 + (N-31)*4 + (N-31)*( 4 + 7*num_states) |

## Impact of Specialized Instructions and Synergistic

The following describe the effects of the specialized operations except for MEM2, MEM4, and NOREG which are handled in the integrated equations.

### Find Max (N-31)

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | MAXL4,5 eliminated | (ALU) -(2*num_states)*(N-31) |
| BPOS | MAXL5 eliminated | (ALU) -num_states*(N-31) |
| ZOL | MAXL4-6 eliminated | (ALU) -3*num_states*(N-31) |

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | MAXL4,5 added back | (ALU) target +(2*num_states)*(N-31) |
| BPOS, BDEC | MAXL5 added back | (ALU) target+ num_states*(N-31) |
| BPOS, ZOL | MAXL5 added back | (ALU) target+ num_states*(N-31) |
| BPOS, BDEC, ZOL | MAXL,5 eliminated (again) | (ALU) target - num_states*(N-31) |

### Traceback Unit (N-31)

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| INDEX | TBU1 eliminated | (ALU) -(N-31) |
| EXTRACT | TBU3 eliminated | (ALU) -(N-31) |

No synergies.

### Add Compare Select (N*num_states)

Subtractive Modifiers

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BPOS | PMUL23 eliminated | (ALU) -N*num_states |

No synergies

### Path Metric Unit (N)

Subtractive Modifiers

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | PMUL22,23 eliminated | (ALU) -N*2*num_states |
| BPOS | PMUL23 eliminated | (ALU) - N*num_states |
| ZOL | PMUL22-24 eliminated | (ALU) +N*(1-3*num_states) |
| VSL | PMUL19 eliminated | (ALU) - N*num_states |
| INDEX | PMUL3,5,9,12,16 | (ALU) -5*N*num_states |

Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | PMUL22,23 added back | (ALU) target +2*N*num_states |
| BPOS, BDEC | PMUL23 added back | (ALU) target+ N*num_states |
| BPOS, ZOL | PMUL23 added back | (ALU) target+ N*num_states |
| BPOS, BDEC, ZOL | PMUL23 eliminated (again) | (ALU) target - N*num_states |

### Hard metric (N*num_states*2)

Subtractive Modifiers

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| EXTRACT | W1 eliminated | (ALU) -N*num_states*2*r |

No synergies

### Branch Metric Unit (N)

Subtractive Modifiers

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | BMUOL1,2 eliminated | (ALU) -N*2*num_states |
| BPOS | BMUOL2 eliminated | (ALU) -N*num_states |
| ZOL | BMUOL1-3 eliminated | (ALU) +N*(1-3*num_states) |

Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | BMUOL1,2 added back | (ALU) target + N*2*num_states |
| BPOS, BDEC | BMUOL2 added back | (ALU) target+ N*num_states |
| BPOS, ZOL | BMUOL2 added back | (ALU) target+ N*num_states |
| BPOS, BDEC, ZOL | BMUOL2 eliminated (again) | (ALU) target - N*num_states |

### Main Decoder

Subtractive

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | INL3,4, MNL6,7, CPYL5,6, FRL6,7 eliminated | (ALU) -2*(num_states-1) -2*N - 2*N*num_states - 31*2 |
| BPOS | INL4, MNL4,7, CPYL6, FRL7 eliminated | (ALU) - N*(num_states-1) - 2*N - N*num_states - 31 |
| ZOL | IN3-5, MNL6-8, CPYL5-7, FRL6-8 eliminated | (ALU) +(1-3*(num_states-1)) + 1-3*N + N*(1-3*num_states) - 31*3 |
| EXTRACT | FRL1-3 eliminated | (ALU) -3*31 |
| INDEX | FR2 eliminated | (ALU) -1 |

Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | INL3,4, MNL6,7, CPYL5,6, FRL6,7 added back | (ALU) target + 2*(num_states-1) +2*N + 2*N*num_states + 31*2 |
| BPOS, BDEC | INL4, MNL7, CPYL6, FRL7 added back | (ALU) target+ N*(num_states-1) + N + N*num_states + 31 |
| BPOS, ZOL | INL4, MNL7, CPYL6, FRL7 added back | (ALU) target+ N*(num_states-1) + N +N*num_states + 31 |
| BPOS, BDEC, ZOL | INL4, MNL7, CPYL6, FRL7 eliminated again | (ALU) target - N*(num_states-1) - N - N*num_states - 31 |

### Integrated Equations

| Class | Equation |
| --- | --- |
| Raw | 13 +(num_states-1)*5 + N*(11 +2 + num_states*(7+7 +24+6.5+ 2*(2+r*2-1))) + 8*31 + (N-31)*(6+4+6*num_states) |
| Memory | 7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states) |
| Multiplication | 0 |
| ALU | 6 + (num_states-1)*3 + N*(8 + 2+6.5+num_states*(3+3+14+2*(2+r*2-1))) +31*(7) +(N-31)*(4 + 5+6*num_states) |

Subtractive Equations

Note that the impact of these modifiers is described in the preceding and not duplicated here for space considerations.

| Instruction | Modifier Equation |
| --- | --- |
| BDEC | (ALU) -(2*num_states)*max((N-31),1)-N*4*num_states -2*(num_states-1) -2*N*(num_states+1) -62 |
| BPOS | (ALU) -num_states*max((N-31),1) - 4*N*num_states -N*max((num_states-1),1) - 2*N – 31 |
| ZOL | (ALU) -3*num_states*max((N-31),1)-9*N*num_states -3*max((num_states-1),1) - 91 |
| EXTRACT | (ALU) -max((N-31),1)-N*num_states*2*r-3*31 |
| INDEX | (ALU) -1-5*N*num_states-max((N-31),1) |
| VSL | (ALU) -N*num_states |
| MEM2 | (MEM) –(7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states))/2 |
| MEM4 | (MEM) -3*(7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states))/4 |
| NOREG | (MEM) -(7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states)) |

Synergistic Modifiers

| Modifiers | Modifier Equation |
| --- | --- |
| BDEC, ZOL | (ALU) target + 2*(N*max((num_states-1),1) + N + 31 +3*N*num_states +num_states*max((N-31),1)) |
| BPOS, BDEC | (ALU) target+ (N*max((num_states-1),1) + N + 31 +3*N*num_states +num_states*max((N-31),1)) |
| BPOS, ZOL | (ALU) target+ (N*max((num_states-1),1) + N + 31 +3*N*num_states +num_states*max((N-31),1)) |
| BPOS, BDEC, ZOL | (ALU) target- (N*max((num_states-1),1) + N + 31 +3*N*num_states +num_states*max((N-31),1)) |
| MEM2, NOREG | (MEM) target + (7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states))/2 |
| MEM4, NOREG | (MEM) target +3*(7 +(num_states-1)*2 + N*(3 + num_states*(4+4 + 10)) + 1*31+(N-31)*(2+1+num_states))/4 |

# Polyphase Interpolator

This component represents the implementation of a block real polyphase interpolator with upsampling factor r, original filter length N, to be calculated for M input samples. N/r is assumed to be an integer. (If it’s not, the designer might as well use more coefficients for less ripple as it’ll take the same amount of time. However, coefficients can be padded.) Note that coef is the address for the coefficients for the original filter, not the polyphase subfilters. Subfiltering is taken care of automatically.

## Pseudocode

Parameters: M (block length of input data)

N (filter length)

R (upconversion rate)

Requires: Circular buffering

y=polyphase_interpolate(coef, data, length, output_array)

//Move input parameters to local registers

1	(instruction to store previous setting in local register)

2	(instruction to turn on circ buff)

3	(instruction to set buffer length)

4	R11 = data

5	R3 = length (actual #)

6	R8 = output_array (address)

//*****************

// block loop

OL1	(outer loop) R2 = data (address) + R5 // block loop

OL2	R7 = r  //number of subfilters

OL3	R10 = coef (address) +  r //points r above h[0]

//*****************

// filter loop

ML1	(middle loop) R1 = R10– R7 //set up pointer to base of appropriate subfilter

ML2	R2 = R11 //reset data pointer

ML3	acc = 0 //zero accumulator (typically done by subtracting a register from itself)

ML4	R9 = N/r  //length of subfilter, hard coded

//*****************

// subfilter loop

IL1	(inner loop) R4 = *R1 //get coefficient p(R10-R7)[k]

IL2	R1 = R1 + N/r – 1 // hard coded value, not two operations // eliminated by indexing

IL3	R5 = *R2++ // note: data steps at normal size

IL4	R6 = R5 * R4

IL5	acc = acc + R6

IL6	R9= R9 – 1 //decrement count

IL7 	flag = Compare R9,0

IL8	If flag, branch inner Loop

//******************

ML5 	*R8++ = acc //store subfilter result

ML6	R7= R7 – 1 //decrement count

ML7 	flag = Compare R7,0

ML8	If flag, branch middle Loop

// end filter loop

//****************

OL4	R11++	//increment data pointer

OL5	R3= R3 – 1 //decrement count

OL6 	flag = Compare R3,0

OL7	If flag, branch outer Loop

// end block loop

//****************

//	Restore stuff

7	(instruction to turn reset addressing mode)

8	(instruction to turn reset buffer length)

9	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 9 + 8*(N/r)*r*M + 8*r*M + 7*M |
| Memory | 2*(N/r)*r*M + 1*r*M + 3*M |
| Multiplication | (N/r)*r*M |
| ALU | 9+ 5*(N/r)*r*M + 7*r*M + 4*M |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | IL7,8, ML6,7, OL5,6 eliminated | (ALU) -2*(N*M +r*M +M) |
| BPOS | IL8, ML7, OL6 eliminated | (ALU) -(N*M +r*M +M) |
| ZOL | IL7-9, ML6-8, OL5-7 eliminated | (ALU) -3*(N*M +r*M +M) |
| MAC | IL5 eliminated | (ALU) -N*M |
| INDEX | IL2 eliminated | (ALU) -N*M |
| MEM2 | MEM operations cut in half | (MEM) -(2*(N/r)*r*M + 1*r*M + 3*M)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(2*(N/r)*r*M + 1*r*M + 3*M)/4 |
| NOREG | MEM operations eliminated | (MEM) - (2*(N/r)*r*M + 1*r*M + 3*M) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | IL7,8, ML6,7, OL5,6 added back | (ALU) target +2*(N*M +r*M +M) |
| BPOS, BDEC | IL8, ML7, OL6 added back | (ALU) target+(N*M +r*M +M) |
| BPOS, ZOL | IL8, ML7, OL6 added back | (ALU) target+(N*M +r*M +M) |
| BPOS, BDEC, ZOL | IL8, ML7, OL6 eliminated (again) | (ALU) target - (N*M +r*M +M) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (2*(N/r)*r*M + 1*r*M + 3*M)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target 3*(2*(N/r)*r*M + 1*r*M + 3*M)/4 |

# AM Modulator

This component represents the implementation of a block AM modulator for DSB-SC AM with a block of length N. The module takes as inputs a pointer to a sine table, an increment factor (to define a carrier frequency), a pointer to a message data block, and an output buffer.

## Pseudocode

Parameters: N (block length of input data)

Requires: Circular buffering

y=AM_modulate(message, sine, increment, offset, output_array, length)

//Move input parameters to local registers

1	(instruction to store previous setting in local register)

2	(instruction to turn on circ buff) // for sine buffer

3	(instruction to set buffer length) // presumably known

4	R1 = message

5	R2 = sine

6	R3 = offset

7	R2 = R2 + R3

8	R3 = increment

9	R4 = length (actual #) //also loop counter

10	R5 = output_array

//*****************

// loop

L1	R6 = *R2 (sine sample)

L2	R2 = R2 + R3

L3	R7 = *R1++ //fetch message

L4	R8 = R6*R7 //modulate

L5	*R5++ = R8 // write to output

L6	R4= R4 – 1 //decrement count

L7 	flag = Compare R4,0

L8	If flag, branch Loop

// end block loop

//****************

//	Restore stuff

11	(instruction to turn reset addressing mode)

12	(instruction to turn reset buffer length)

13	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 13 + 8*N |
| Memory | 3*N |
| Multiplication | N |
| ALU | 13 + 4*N |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L6,7 eliminated | (ALU) -2*N |
| BPOS | L7 eliminated | (ALU) -N |
| ZOL | L6-8 eliminated | (ALU) 1-3*N |
| MAC | L5 eliminated | (ALU) –N |
| INDEX | L2 eliminated | (ALU) –N |
| MEM2 | MEM operations cut in half | (MEM) – (3*N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(3*N)/4 |
| NOREG | MEM operations eliminated | (MEM) – 3*N |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L6,7 added back | (ALU) target + 2*N |
| BPOS, BDEC | L7 added back | (ALU) target + N |
| BPOS, ZOL | L7 added back | (ALU) target + N |
| BPOS, BDEC, ZOL | L7 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (3*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(3*N)/4 |

# AM Demodulator

This component represents the implementation of a simple Costas-loop PLL for DSB-SC AM. The module takes as inputs a pointer to a sin/cos function, a phase accumulator, I and Q branch accumulators, and an output_array, and a message data block. All low-pass filters are implemented as integrators. Note a slightly different structure is needed for samples generated from a complex ADC.

## Pseudocode

Parameters: N (block length of input data)

Ovsp_factor (ratio of carrier to sampling rate)

C cycles to call sin function

y=AM_demodulate(message, sin_function, phase_acc, phase_inc, I_acc, Q_acc, output_array, length)

//Move input parameters to local registers

1	R1 = message

2	R2 = sin_function // function to call

//	R3 = sin_val

//	R4 = cos_val

3	R5 = length (actual #) //also loop counter

4	R6 = output

5	R7 = phase_acc

6	R8 = I_acc

7	R9 = Q_acc

8	R14 = phase_inc

//*****************

// loop

L1	R6 = *R1++

//	Generate

L2	(write R7 to appropriate function register)

LC	call sin_function // puts sin_val + cos_val into R3, and R4 (branch R2)

L3	R10 = R4*R1 //	Generate I branch

L4	R8 = R8 + R10 // LPF acc

L5	R11 = R3*r1 //	Generate Q branch

L6	R9 = R9 + R11 // LPF acc

L7	*R6++ = R8 //output I branch LPF as message

L8	R12 = R8*R9 //Generate error term

L9 	R7 = R7 + R12 // LPF error term acc

L10	R13 = R7 + R14 //phase_inc

L11	flag = cmp(R13, (value for 2pi)

L12	if flag (R13 < 2pi) branch L14

L13.25	R13=R13 – (value for 2pi) //assume oversampling factor of 4 for labeling purposes

L14	R5= R5 – 1 //decrement bit count

L15 	flag = Compare R5,0

L16	If flag, branch Loop

// end block loop

//****************

//	Restore stuff

9	(instruction to store I_acc)

10	(instruction to store Q_acc)

11	(instruction to store phase)

12	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 11+ N*15 + N/ Ovsp_factor + C*N |
| Memory | 2*N |
| Multiplication | 3*N |
| ALU | 11+ 10*N + N/Ovsp_factor |
| Function | N*C (doesn’t get hit by VLIW or SIMD) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L14,15 eliminated | (ALU) -2*N |
| BPOS | L15 eliminated (no good way to eliminate L12) | (ALU) -N |
| ZOL | L14-16 eliminated | (ALU) 1-3*N |
| MAC | L4,6,9 eliminated | (ALU) – 3*N |
| COND_EXEC | L13.25 eliminated | (ALU) –N/Ovsp_factor |
| MEM2 | MEM operations cut in half | (MEM) – (2*N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(2*N)/4 |
| NOREG | MEM operations eliminated | (MEM) – (2*N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L14,15 added back | (ALU) target + 2*N |
| BPOS, BDEC | L15 added back | (ALU) target + N |
| BPOS, ZOL | L15 added back | (ALU) target + N |
| BPOS, BDEC, ZOL | L15 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(2*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target +3*(2*N)/4 |

# FM Modulator

This component represents the implementation of a block FM modulator with a block of length N. The module takes as inputs a pointer to a sine table, an increment factor (to define a carrier frequency), a pointer to a message data block, and an output buffer. The frequency deviation constant is built in rather than being passed in (difference of a cycle). Note that in practical implementations, this would be coupled with a pre-emphasis filter (which adds gain at higher frequencies relative to lower frequencies). Also note that the increment here refers to the phase step for the carrier.

## Pseudocode

Parameters: 	N (block length of input data)

Ovsp_factor (ratio of carrier to sampling rate)

C: trig function time

y=FM_modulate(message, sin_function, message_acc, phase_inc, output_array, length)

//Move input parameters to local registers

1	R1 = message

2	R2 = sine_function

3	R3 = message_acc

4	R5 = phase_inc

5	R6 = length (actual #) //also loop counter

6	R7 = output_array

//*****************

// loop

L1	R8 = *R1++ //get message value

L2	R3 = R3 + R8 //accumulate message // in the current form this should be forced to have zero DC  // bias, otherwise some extra cycles will be needed to make the abs of this value < 2pi

L3	R4 = R3 * scale // multiply by frequency deviation constant

L4	R4 = R4 + phase_inc //not an accumulate (could be done as a MAC, but then accumulator would

// have to be reset every time through)

L5	flag = Compare R4 < (value for 2 pi)

L6	If flag, branch L7

L6.25	R4 = R4 – (value for 2 pi)

//	Generate modulated

L7	(write R4 to appropriate function register)

LC	call sin_function // puts sin_val into R9

L8	*R7++ = R9 //write output

L9	R6 = R6 – 1 //decrement block count

L10 	flag = Compare R6,0

L11	If flag, branch Loop

//****************

//	Restore stuff

7	(store message_acc)

8	(nstruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 8+ N*11 + N/ Ovsp_factor + C*N |
| Memory | 2*N |
| Multiplication | N |
| ALU | 8+ 9*N + N/Ovsp_factor |
| Function | N*C (doesn’t get hit by VLIW or SIMD) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L9,10 eliminated | (ALU) -2*N |
| BPOS | L10 eliminated (no good way to eliminate L6) | (ALU) -N |
| ZOL | L9-11 eliminated | (ALU) 1-3*N |
| COND_EXEC | L6.25 eliminated | (ALU) -N/Ovsp_factor |
| MEM2 | MEM operations cut in half | (MEM) - (2*N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(2*N)/4 |
| NOREG | MEM operations eliminated | (MEM) - (2*N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L9,10 added back | (ALU) target + 2*N |
| BPOS, BDEC | L10 added back | (ALU) target + N |
| BPOS, ZOL | L10 added back | (ALU) target + N |
| BPOS, BDEC, ZOL | L10 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (2*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target +3*(2*N)/4 |

# FM Demodulator

This component represents the implementation of a block FM demodulator with a block of length N. The module takes as inputs a pointer to an arctangent function, I and Q samples, stored loop and phase accumulators, length, and the output array. Note that complex input samples could possibly be created via an external Hilbert transform or from a complex ADC.

## Pseudocode

Parameters: 	N (block length of input data)

Ovsp_factor (ratio of carrier to sampling rate)

C: trig function time

FM_demodulate(sig_I, sig_Q, arctan_function, loop_acc, phase_acc, output_array, length, sin_function)

//Move input parameters to local registers

1	R1 = sig_I

2	R2 = sig_Q

3	R3 = length

4	R4 = output_array

5	R5 = phase_acc

6	R6 = loop_acc

//*****************

// loop

//generate appropriate real, imag from

L1	(push phase_acc to appropriate register for sin_cos call)

LC1	sin_cos_function // function to call, put in R7,R8

//complex multiplication

//	I

L2	R9 = R1 * R7

L3	R11 = R2*R8

L4	R9 = R9 – R11

//	Q

L5	R10 = R2 * R7

L6	R11 = R1 * R8

L7	R10 = R10 + R11

L8	(push R9 to appropriate register for atan call)

L9	(push R10 to appropriate register for atan call)

LC2	atan_function (assume ends up in R7)

L10	R8 = R7 * loop_constant //needed to tweak loop BW

L11	R6 = R6 + R8//accumulate loop

L12	R7 = R7 + R6 //actual output

L13	*R4++ = R7

//phase branch

L14	R8 = R7 * a different constant

L15	R5 = R5 + R8 //phase accumulate

L16	R5 = R5 + hard_code_carrier_step

L17	flag = Compare R5 < (value for 2 pi)

L18	If flag, branch L19

L18.25	R5 = R5 – (value for 2 pi)

L19	R3 = R3 – 1 //decrement block count

L20 	flag = Compare R3,0

L21	If flag, branch Loop

// end block loop

//****************

//	Restore stuff

7	(instruction to store phase_acc)

8	(instruction to store loop_acc)

9	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 9+ N*(21+C1+C2) + N/ Ovsp_factor |
| Memory | 4*N |
| Multiplication | 6*N |
| ALU | 9+ 11*N + N/Ovsp_factor |
| Function | N*(C1+C2) (doesn’t get hit by VLIW or SIMD) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L19,20 eliminated | (ALU) -2*N |
| BPOS | L20 eliminated (no good way to eliminate L6) | (ALU) -N |
| ZOL | L19-21 eliminated | (ALU) 1-3*N |
| MAC | L4,7,11,15 | (ALU) – 4*N |
| COND_EXEC | L18.25 eliminated | (ALU) –N/Ovsp_factor |
| MEM2 | MEM operations cut in half | (MEM) – (4*N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(4*N)/4 |
| NOREG | MEM operations eliminated | (MEM) – (4*N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L19,20 added back | (ALU) target + 2*N |
| BPOS, BDEC | L20 added back | (ALU) target + N |
| BPOS, ZOL | L20 added back | (ALU) target + N |
| BPOS, BDEC, ZOL | L20 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (4*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target +3*(4*N)/4 |

# BPSK Modulator

This component represents the implementation of a block BPSK sine wave modulator. T modulates N 16-bit words with M samples per symbol. Sine values are generated by stepping through a sine table and phase shifts accomplished by stepping half way through the sine table. To simplify the implementation, the sine wave buffer is set up for circular addressing.

## Pseudocode

Parameters: N (number input words – 16-bit)

M (samples / symbol)

Requires: Circular buffering

BPSK_mod(word_ptr, sine_table, output_buffer, increment)

//Move input parameters to local registers

1	(instruction to store previous settings)

2	(instruction to store pervious settings)

3	(instruction to turn on circ buff) (for sine_table)

4	(instruction to set buffer length)

5	R1 = word_ptr

6	R2 = sine_table

7	R3 = output_buffer

8	R4 = N

9	R5 = increment //(defines frequency)

10	R6 = 0 //used to store old bit

//*****************

// word loop

OL1	(outer loop) R7 = *R1++ //fetch word

OL2	R8= 16 // set bit counter

//*****************

// bit loop

ML1	R9 = R7 &1

ML2	R7 = R7 >> 1

ML3	R10 = CMPEQ (R9, R6)

ML4	if !R10 branch ML6

ML5	R2 = R2 + pi (equivalent in index)

ML6	R11 = M

ML7 	R6 = R9

//*****************

// sine loop

IL1	R12 = *R2

IL2	R2 = R2 + R5 //indexed addressing

IL3	*R3++ = R12

IL4	R11= R11 – 1 //decrement bit count

IL5 	flag = Compare R11,0

IL6	If flag, branch sine Loop

// END SINE LOOP

//*******************

ML8	R8= R8 – 1 //decrement bit count

ML9 	flag = Compare R8,0

ML10	If flag, branch bit Loop

// END BIT LOOP

//*******************

OL3	R4= R4 – 1 //decrement word count

OL4 	flag = Compare R4,0

OL5	If flag, branch word Loop

// END WORD LOOP

//*******************

//Restore stuff

11	(instruction to reset addressing mode)

12	(instruction to reset buffer settings)

13	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 13 + 6*M*16*N + 16*N*10 + 5*N |
| Memory | 2*M*16*N+N |
| Multiplication | 0 |
| ALU | 13+ 4*M*16*N + 16*N*10+4*N |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL3,4, ML9,10, IL5,6 eliminated | (ALU) -2*(M*16*N + 16*N+N) |
| BPOS | OL4, ML10, IL6 eliminated | (ALU) -(M*16*N + 16*N+N) |
| ZOL | L19-21 eliminated | (ALU) -3*(M*16*N + 16*N+N) |
| EXTRACT | ML2 eliminated | (ALU) -16*N |
| INDEX | IL2 eliminated | (ALU) -M*16*N |
| MEM2 | MEM operations cut in half | (MEM) - (2*M*16*N+N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(2*M*16*N+N)/4 |
| NOREG | MEM operations eliminated | (MEM) - (2*M*16*N+N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL3,4, ML9,10, IL5,6 added back | (ALU) target + 2*(M*16*N + 16*N+N) |
| BPOS, BDEC | OL4, ML10, IL6 added back | (ALU) target + (M*16*N + 16*N+N) |
| BPOS, ZOL | OL4, ML10, IL6 added back | (ALU) target + (M*16*N + 16*N+N) |
| BPOS, BDEC, ZOL | OL4, ML10, IL6 eliminated (again) | (ALU) target - (M*16*N + 16*N+N) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (2*M*16*N+N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(2*M*16*N+N)/4 |

# BPSK Demodulator

This component represents the implementation of a block BPSK demodulator. Specifically, it maps signal levels (presumably from the output of a symbol-synchronization process) to bits and packs them into 16-bit words. Note that this block does not do either symbol or carrier synchronization. For high SNR environments, the AM demodulator can be used to implement a BPSK PLL.

## Pseudocode

Parameters: N (number input samples – assumed to be divisible by 16)

BPSK_demod(input_data, output_buffer, N)

//Move input parameters to local registers

1	R1 = input_data

2	R2 = output_buffer

3	R3 = N

4	R4 = 16

5	R5 = 0

6	R6 = 2^15 (1000 0000 0000 0000)

//*****************

// sample

L1	(loop) R7 = *R2++ //fetch sample

L2	if R7<0 branch L4

L2,5	R5 = R5 XOR R6 //should happen half the time

L3	R6 = R6 >> 1 //used to set where inputs go in (0 fill)

L4	R4 = R4-- //decrement bit counter

L5	flag = cmpgt (R4,0) //note that using conditional execution/moving of the following would actually

// ADD cycles because the instructions would be fetched every time instead of just once

L6	if flag (R4 >0), branch L11

L6.1	*R2++ = R5 //write filled word happens once every 16 passes

L6.2	R4 = 16 // reset bit counter

L6.3	R6 = 2^15

L7	R3= R3 – 1 //decrement sample count

L8 	flag = cmpgt R3,0

L9	If flag (R3>0), branch Loop

7	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 7 + N*(9+0.5+3/16) |
| Memory | N*(1+1/16) |
| Multiplication | 0 |
| ALU | 7 + N*(8+0.5+2/16) |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | L7,8 eliminated | (ALU) -2*N |
| BPOS | L5,8 eliminated | (ALU) –2*N |
| ZOL | L7-9 eliminated | (ALU) 1-3*N |
| COND_EXEC | L2.5 eliminated | (ALU) -0.5*N |
| MEM2 | MEM operations cut in half | (MEM) – (N*(1+1/16))/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(N*(1+1/16))/4 |
| NOREG | MEM operations eliminated | (MEM) – (N*(1+1/16)) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | L7,8 added back | (ALU) target + 2*N |
| BPOS, BDEC | L8 added back | (ALU) target + N |
| BPOS, ZOL | L8 added back | (ALU) target + N |
| BPOS, BDEC, ZOL | L8 eliminated (again) | (ALU) target - N |
| MEM2, NOREG | MEM2 effect undone | (MEM) target + (N*(1+1/16))/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N*(1+1/16))4 |

# BFSK Modulator

This component represents the implementation of a block binary frequency shift keying (BFSK) modulator leveraging a sine lookup table with two different increment offsets (corresponding to two different frequencies). Note that this approach eliminates the need for any special relationship between symbol length and encoding frequencies because there are no abrupt phase transition between symbols. The block length is N 16-bit words with FSK symbols with M samples. M is assumed to be hardcoded (though this is not too important).

## Pseudocode

Parameters: N (# 16 bit words to encode)

M (symbol length)

Requires: Circular buffering (for wrap around in sine table)

BFSK_encode(word_ptr, sine_table, output_buffer, increment1, increment2, N)

//Move input parameters to local registers

1	(instruction to store previous settings)

2	(instruction to store pervious settings)

3	(instruction to turn on circ buff) (for sine_table)

4	(instruction to set buffer length)

5	R1 = word_ptr

6	R2 = sine_table

7	R3 = output_buffer

8	R4 = N

9	R5 = increment1 //(defines frequency 1)

10	R14 = increment2 //(defines frequency 2)

//*****************

// word loop

OL1	(outer loop) R7 = *R1++ //fetch word

OL2	R8= 16 // set bit counter

//*****************

// bit loop

ML1	R9 = R7 &1

ML2	R7 = R7 >> 1

ML3	flag = CMPEQ (R9, 0)

ML4	if flag branch ML6

ML5	R6 = R5 //increment = increment 1 // note one of the ML5s must execute

ML5.5	branch ML8

ML5	R6 = R14 // increment = increment2

ML6	R11 = M // no particular reason that this divide evenly into the length of the sine table

//*****************

// sine loop

IL1	R12 = *R2

IL2	R2 = R2 + R6 //indexed addressing

IL3	*R3++ = R12

IL4	R11= R11 – 1 //decrement bit count

IL5 	flag = Compare R11,0

IL6	If flag, branch sine Loop

// END SINE LOOP

//*******************

ML7	R8= R8 – 1 //decrement bit count

ML8 	flag = Compare R8,0

ML9	If flag, branch bit Loop

// END BIT LOOP

//*******************

OL3	R4= R4 – 1 //decrement word count

OL4 	flag = Compare R4,0

OL5	If flag, branch word Loop

// END WORD LOOP

//*******************

//Restore stuff

11	(instruction to reset addressing mode)

12	(instruction to reset buffer settings)

13	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 13+5*N+9.5*N*16 + 6*M*N*16 |
| Memory | N+2*M*N*16 |
| Multiplication | 0 |
| ALU | 13+4*N+9.5*N*16+4*M*N*16 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL3,4,ML7,8, IL4,5 eliminated | (ALU) -2*(N+N*16+M*N*16) |
| BPOS | OL4,ML3,8, IL5 eliminated | (ALU) –(N+2*N*16+M*N*16) |
| ZOL | OL3-5,ML7-9, IL4-6 eliminated | (ALU) -3*(N+N*16+M*N*16) |
| COND_EXEC | ML5.5 eliminated (that’s the effect at least) | (ALU) -0.5*N |
| EXTRACT | ML2 eliminated | (ALU) -16*N |
| INDEX | IL2 eliminated | (ALU) -M*16*N |
| MEM2 | MEM operations cut in half | (MEM) - (N+2*M*N*16)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(N+2*M*N*16)/4 |
| NOREG | MEM operations eliminated | (MEM) - (N+2*M*N*16) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL3,4,ML7,8, IL4,5 added back | (ALU) target + 2*(M*16*N + 16*N*+N) |
| BPOS, BDEC | OL4,ML8, IL5 added back | (ALU) target + (M*16*N + 16*N*+N) |
| BPOS, ZOL | OL4,ML8, IL5 added back | (ALU) target + (M*16*N + 16*N*+N) |
| BPOS, BDEC, ZOL | OL4,ML8, IL5 eliminated (again) | (ALU) target - (M*16*N + 16*N*+N) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(N+2*M*N*16)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N+2*M*N*16)/4 |

# BFSK Demodulator

This component represents the implementation of a noncoherent block binary frequency shift keying (BFSK) demodulator. This is done by passing the received signal through two filters tuned to the two frequencies with the output of one filter subtracted from another. This would then need to be passed through a symbol-synchronization circuit to create actual output samples. This in turn should be passed through the BPSK demodulator defined previously to create bits and packed words. Note that because this is a block operation, we’re assuming alignment occurs external to this procedure.

## Pseudocode

Parameters: N (# samples)

Filt_length (filter length)

Requires: Circular buffering (for filtering)

BFSK_decode(signal, coef1, coef2, output_buffer,  N, filt_length)

//Move input parameters to local registers

1	(instruction to store previous settings)

2	(instruction to store pervious settings)

3	(instruction to store previous settings)

4	(instruction to store pervious settings)

5	(instruction to turn on circ buff) (for coef1)

6	(instruction to set buffer length)

7	(instruction to turn on circ buff) (for coef2)

8	(instruction to set buffer length)

//preceding eliminates need to reset R2, R3 pointers

9	R1 = signal

10	R2 = coef1

11	R3 = coef2

12	R4 = N

13	R10 = output_buffer

//OUTER LOOP

//Set up FILTERS

OL1	acc1 = 0

OL2	acc2 = 0

OL3	R9 = filt_length //hard coded

/

//Inner Loop (Filters)

IL1	R4 = *R1++ //data

IL2	R5 = *R2++ //filt1

IL3	R6 = R5 * R4

IL4	acc1 = acc1 + R6

IL5	R5 = *R3++ //filt2

IL6	R6 = R5 * R4

IL7	acc2 = acc2 + R6

IL8	R9 = R9 – 1 //decrement block count

IL9 	flag = Compare R9,0

IL10	If flag, branch inner Loop

//END FILTERS LOOP

OL4	R11 = acc1 – acc2

OL5	*R10++ = R11

OL5	R1 = R1 – filt_length-1 //scaled as need be for word size

OL6	R4 = R4 – 1 //decrement block count

OL7 	flag = Compare R4,0

OL8	If flag, branch Loop

// END WORD LOOP

//*******************

//Restore stuff

14	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 14 + N*8 + N*filt_length*10 |
| Memory | N +3*filt_length*N |
| Multiplication | 2*filt_length*N |
| ALU | 14 + N*7 + 5*N*filt_length |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL6,7,IL8,9 eliminated | (ALU) -2*(N+N*filt_length) |
| BPOS | OL7, IL9 eliminated | (ALU) –(N+N*filt_length) |
| ZOL | OL6-8,IL8-10 eliminated | (ALU) -3*(N+N*filt_length) |
| MAC | IL4,7 eliminated | (ALU) -2*N*filt_length |
| MEM2 | MEM operations cut in half | (MEM) - (N +3*filt_length*N)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(N +3*filt_length*N)/4 |
| NOREG | MEM operations eliminated | (MEM) - (N +3*filt_length*N) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL6,7, IL8,9 added back | (ALU) target + 2*(N+N*filt_length) |
| BPOS, BDEC | OL7, IL9 added back | (ALU) target + (N+N*filt_length) |
| BPOS, ZOL | OL7, IL9 added back | (ALU) target + (N+N*filt_length) |
| BPOS, BDEC, ZOL | OL7, IL9 eliminated (again) | (ALU) target - (N+N*filt_length) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(N +3*filt_length*N)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N +3*filt_length*N)/4 |

# 16-QAM Modulator

This component represents the implementation of a 16-QAM modulator. It takes in 16-bit words and maps this to I and Q values. An additional routine would be needed to modulate these I and Q values onto a carrier.

## Pseudocode

Parameters: N (# words)

Requires:

16_QAM_mod(signal, out_I, out_Q, LUT_real, LUT_imag, length)

//Move input parameters to local registers

1	R1 = signal

2	R2 = out_I

3	R3 = out_Q

4	R4 = LUT_real

5	R5 = LUT_imag

6	R10 = length

//OUTER LOOP

OL1	R6 = *R1++

OL2	R11 = 4

//INNER LOOP

IL1	R7 = R6  >> 2 // GET 2 bits

IL2	R7 = R7 & 3

IL3	R8 = R4 + R7 //LUT_REAL

IL4	R9 = *R8

IL5	*R2 = R9 //WRITE OUTPUT

IL6	R7 = R6  >> 2 // GET 2 bits

IL7	R7 = R7 & 3

IL8	R8 = R5 + R7 //LUT_IMAG

IL9	R9 = *R8

IL10	*R3 = R9 //WRITE OUTPUT

IL11	R11 = R11 – 1 //decrement block count

IL12 	flag = Compare R4,0

IL13	If flag, branch Inner Loop

//END INNER LOOP

OL3	R10= R10 – 1 //decrement block count

OL4 	flag = Compare R4,0

OL5	If flag, branch Loop

// END WORD LOOP

//*******************

//Restore stuff

7	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 7 + N*5 + N*4*13 |
| Memory | N +N*4*4 |
| Multiplication | 0 |
| ALU | 7 + N*4 + N*4*9 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL3,4,IL11,12 eliminated | (ALU) -2*(N + 4*N) |
| BPOS | OL4, IL12 eliminated | (ALU) -(N + 4*N) |
| ZOL | OL3-5,IL11-13 eliminated | (ALU) -3*(N + 4*N) |
| EXTRACT | IL1,6 eliminated | (ALU) -2*4*N |
| INDEX | IL3,8 eliminated | (ALU) -2*4*N |
| MEM2 | MEM operations cut in half | (MEM) - (N +N*4*4)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) - 3*(N +N*4*4)/4 |
| NOREG | MEM operations eliminated | (MEM) - (N +N*4*4) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL3,4,IL11,12 added back | (ALU) target + 2*(N + 4*N) |
| BPOS, BDEC | OL4,4,IL12 added back | (ALU) target + (N + 4*N) |
| BPOS, ZOL | OL4,4,IL12 added back | (ALU) target + (N + 4*N) |
| BPOS, BDEC, ZOL | OL4,4,IL12 re-elimianted | (ALU) target - (N + 4*N) |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(N +N*4*4)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N +N*4*4)/4 |

# 16-QAM Demodulator

This component represents the implementation of a 16-QAM demodulator. It takes in I/Q samples, maps these to bits and packs these into 16-bit words. Note separate processes would be needed for carrier recovery, channel equalization, and symbol synchronization.

## Pseudocode

Parameters: N (# samples)

16_QAM_demod(signal_I, signal_Q, out, length)

//Move input parameters to local registers

1	R1 = signal_I

2	R2 = signal_Q

3	R3 = out

4	R4 = length

// OUTER LOOP (16 samples per pass)

OL1	R5 = 0

OL2	R11 = 4 // assuming 16-bit words

//INNER LOOP (4 samples per pass)

//I

IL1	R6 = *R1++

IL2	flag = compare R6 < 0

IL3	If flag branch IL5

IL3.5	R5 = R5 + 1

IL4	R5 = R5 << 1

IL5	R6 = abs(R6)

IL6	flag = compare R6 < thresh

IL7	If flag branch IL8

IL7.5	R5 = R5 + 1

IL8	R5 = R5 << 1

//Q

IL9	R6 = *R2++

IL10	flag = compare R6 < 0

IL11	If flag branch IL12

IL11.5	R5 = R5 + 1

IL12	R5 = R5 << 1

IL13	R6 = abs(R6)

IL14	flag = compare R6 < thresh

IL15	If flag branch IL16

IL15.5	R5 = R5 + 1

IL16	R5 = R5 << 1

IL17	R11 = R11 – 1 //decrement block count

IL18 	flag = Compare R11,0

IL19	If flag, branch Inner Loop

// END INNER LOOP

OL3	*R3++ = R5 //store output word

OL4	R10= R10 – 1 //decrement block count

OL5 	flag = Compare R4,0

OL6	If flag, branch Loop

// END OUTER LOOP

//*******************

//Restore stuff

5	(instruction to branch back)

## Raw Operations

| Class | Equation |
| --- | --- |
| Raw | 5 + N*6/16 + N*(19 + 4*0.5)/4 |
| Memory | N/16 + N*(2)/4 |
| Multiplication | 0 |
| ALU | 5 + N*5/16 + N*4*19/4 |

## Impact of Specialized Instructions

| Instruction | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC | OL4,5,IL17,18 eliminated | (ALU) -2*(N/16+ N/4) |
| BPOS | OL5, IL3,7,11,15,18 eliminated | (ALU) –(N/16 + 5*N/4) |
| ZOL | OL4-5,IL17-19 eliminated | (ALU) -3*(N/16 + N/4) |
| COND_EXEC | IL 3.5,7.5,11.5,15.5 eliminated | (ALU) –N/2 |
| VSL | IL4,8,12,16,3.5,7.5,11.5,15.5 | (ALU) –N*(4+4*0.5)/4 |
| MEM2 | MEM operations cut in half | (MEM) – (N/16 +N/2)/2 |
| MEM4 | MEM operations cut to quarter | (MEM) – 3*(N/16 +N/2)/4 |
| NOREG | MEM operations eliminated | (MEM) – (N/16 +N/2) |

## Synergistic Modifiers

| Modifiers | Impact | Modifier Equation |
| --- | --- | --- |
| BDEC, ZOL | OL4,5,IL17,18 added back | (ALU) target + 2*(N/16+ N/4) |
| BPOS, BDEC | OL5,IL18 added back | (ALU) target + (N/16+ N/4) |
| BPOS, ZOL | OL5,IL18 added back | (ALU) target + (N/16+ N/4) |
| BPOS, BDEC, ZOL | OL5,IL18 eliminated (again) | (ALU) target - (N/16+ N/4) |
| COND_EXEC, VSL | IL 3.5,7.5,11.5,15.5 added back | (ALU) target + N/2 |
| MEM2, NOREG | MEM2 effect undone | (MEM) target +(N/16 +N/2)/2 |
| MEM4, NOREG | MEM4 effect undone | (MEM) target + 3*(N/16 +N/2)/4 |

The specific tradeoff is an expected 7.5 * 8 or 60 cycles for specifying # of 16-bit words versus an added 2 cycles per bit (actually a little bit more) for specifying # bits because of the need for extra loop control.