DSP Mappings (Task 1.3a)

for the

Tool for Automating Estimation of DSP Resource Statistics for Waveform Components

Submitted under Subcontract FP-19738-430292

An Integrated Tool for SCA Waveform Development, Testing, and Debugging and a Tool for Automated Estimation of DSP Resource Statistics for Waveform Components

Version 1.1

Revision History

| Version | Summary of Changes | Date |
| --- | --- | --- |
| 0.1 (JN) | Internal Release | 2/23/08 |
| 1.0 (JN) | Release as initial documentation for DSP files | 3/26/08 |
| 1.1 (JN) | Validation errors fixed: C55 (2 ops/cycle) | 5/15/08 |

TABLE OF CONTENTS

1	Introduction and Methodologies	1

1.1	Methodology	1

1.2	Data Sources	1

1.3	Characteristics	3

1.4	Modifiers	3

2	TMS320C6205-200	6

2.1	Information from General Conventions	6

2.2	Information from DataSheet	6

2.3	Peak Power Calculation	6

2.4	Characteristics	6

2.5	Modifiers	6

2.6	VLIW Information	6

2.7	SIMD Information	7

2.8	Benchmarked Routines [4]	7

2.9	References	8

3	TMS320C6713B-300	8

3.1	Information from General Conventions	8

3.2	Information from DataSheet [1]	8

3.3	Peak Power Calculation	8

3.4	Characteristics	8

3.5	Modifiers	9

3.6	VLIW Information	9

3.7	SIMD Information	9

3.8	Benchmarked Routines [4]	9

3.9	References	10

4	TMS320C6455-850	10

4.1	Information from General Conventions	10

4.2	Information from DataSheet [1]	10

4.3	Peak Power Calculation	11

4.4	Characteristics	11

4.5	Modifiers	11

4.6	SIMD Information	12

4.7	Benchmarked Routines [4]	13

4.8	References	13

5	TMS320C6416T-850	13

5.1	Information from General Conventions	14

5.2	Information from DataSheet [1]	14

5.3	Peak Power Calculation	14

5.4	Characteristics	14

5.5	Modifiers	14

5.6	VLIW Information	15

5.7	SIMD Information	16

5.8	Benchmarked Routines [4]	16

5.9	References	16

6	TMS320VC5502-300	17

6.1	Information from General Conventions	17

6.2	Information from DataSheet [1]	17

6.3	Peak Power Calculation	17

6.4	Characteristics	17

6.5	Modifiers	18

6.6	VLIW Information	19

6.7	SIMD Information	19

6.8	Benchmarked Routines [4]	19

6.9	References	20

7	TMS320VC5416-160	20

7.1	Information from General Conventions	20

7.2	Information from DataSheet [1]	20

7.3	Peak Power Calculation	21

7.4	Characteristics	21

7.5	Modifiers	21

7.6	VLIW Information	22

7.7	SIMD Information	22

7.8	Benchmarked Routines [4]	22

7.9	References	23

8	ADSP-BF532	23

8.1	Information from General Conventions	23

8.2	Information from DataSheet [1]	23

8.3	Peak Power Calculation	23

8.4	Characteristics	24

8.5	Modifiers	24

8.6	VLIW Information	25

8.7	SIMD Information	25

8.8	Benchmarked Routines [4]	25

8.9	References	25

9	ADSP-21632	26

9.1	Information from General Conventions	26

9.2	Information from DataSheet [1]	26

9.3	Peak Power Calculation	26

9.4	Characteristics	26

9.5	Modifiers	26

9.6	VLIW Information	27

9.7	SIMD Information	28

9.8	Benchmarked Routines [4]	28

9.9	References	28

10	ADSP-TS201S	28

10.1	Information from General Conventions	28

10.2	Information from DataSheet [1]	28

10.3	Peak Power Calculation	29

10.4	Characteristics	29

10.5	Modifiers:	29

10.6	VLIW Information	31

10.7	SIMD Information	31

10.8	Benchmarked Routines [4]	31

10.9	References	31

11	ADSP-2191M	32

11.1	Information from General Conventions	32

11.2	Information from DataSheet [1]	32

11.3	Peak Power Calculation	32

11.4	Characteristics	32

11.5	Modifiers	33

11.6	VLIW Information	33

11.7	SIMD Information	33

11.8	Benchmarked Routines [4]	33

11.9	References	34

12	Pentium IV 661	34

13	MPC8540	34

13.1	Information from General Conventions	34

13.2	Information from DataSheet [1-5]	34

13.3	Peak Power Calculation	35

13.4	Characteristics	35

13.5	Modifiers	35

13.6	VLIW Information	36

13.7	SIMD Information	36

13.8	Benchmarked Routines	36

13.9	References	37

14	DSP56321	37

14.1	Information from General Conventions	37

14.2	Information from DataSheet [1],[2]	37

14.3	Peak Power Calculation	37

14.4	Characteristics	38

14.5	Modifiers	38

14.6	VLIW Information	39

14.7	SIMD Information	39

14.8	Benchmarked Routines	39

14.9	References	39

15	DSP56F8367	39

15.1	Information from General Conventions	39

15.2	Information from DataSheet [1]	40

15.3	Peak Power Calculation	40

15.4	Characteristics	40

15.5	Modifiers	41

15.6	VLIW Information	41

15.7	SIMD Information	41

15.8	Benchmarked Routines	41

15.9	References	41

16	MSC7116	42

16.1	Information from General Conventions	42

16.2	Information from DataSheet [1]	42

16.3	Peak Power Calculation	42

16.4	Characteristics	42

16.5	Modifiers	43

16.6	VLIW Information	44

16.7	SIMD Information	44

16.8	Benchmarked Routines	44

16.9	References	44

17	MSC8101	44

17.1	Information from General Conventions	44

17.2	Information from DataSheet [1]	44

17.3	Peak Power Calculation	45

17.4	Characteristics:	45

17.5	Modifiers	45

17.6	VLIW Information	46

17.7	SIMD Information	46

17.8	Benchmarked Routines	46

17.9	References	47

18	ARM 968E-S Data Sheet	47

18.1	Information from General Conventions	47

18.2	Information from DataSheet [1]	47

18.3	Peak Power Calculation	47

18.4	Characteristics	47

18.5	Modifiers	48

18.6	VLIW Information	48

18.7	SIMD Information	48

18.8	Benchmarked Routines	48

18.9	References	48

19	ARM 1020E Data Sheet	49

19.1	Information from General Conventions	49

19.2	Information from DataSheet [1]	49

19.3	Peak Power Calculation	49

19.4	Characteristics	49

19.5	Modifiers	50

19.6	VLIW Information	50

19.7	SIMD Information	50

19.8	Benchmarked Routines	50

19.9	References	51

20	ARM1136J(F)-S Data Sheet	51

20.1	Information from General Conventions	51

20.2	Information from DataSheet [1]	51

20.3	Peak Power Calculation	51

20.4	Characteristics	52

20.5	Modifiers	52

20.6	VLIW Information	52

20.7	SIMD Information	53

20.8	Benchmarked Routines	53

20.9	References	53

21	ARM1136J-S	53

21.1	Information from General Conventions	53

21.2	Information from DataSheet [1]	53

21.3	Peak Power Calculation	54

21.4	Characteristics	54

21.5	Modifiers	54

21.6	VLIW Information	55

21.7	SIMD Information	55

21.8	Benchmarked Routines	55

21.9	References	55

# Introduction and Methodologies

This document is intended to document the steps used to generate the DSP files for the “Tool for Automating Estimation of DSP Resource Statistics for Waveform Components” and to provide enough detail for others to be able to create similar files for their processors.

This section gives an overview of the methodology for writing a DSP file and details on key processes associated with this methodology. This is followed by 20 example applications of this process to a variety of different processors.

## Methodology

A DSP file is generated by first collecting the following documentation on the processors:

- Datasheet

- Instruction set

- Architectural documentations

- Peak power consumption

- Vendor provided library code

These documents are then used to determine the following parameters:

- processor clock rate,

- native bit-field width,

- peak power consumption,

- characteristics

- support for VLIW/superscalar instructions,

- support for SIMD,

- specialized instructions which permit multiple operations in a single instruction-cycle, and the existence of vendor provided library code.

With this information, a DSP file for the processor can be written according to the format specified in ]. These steps are then documented for each processor and then validated using mapped components and vendor measured statistics.

This project is mapping the 20 DSPs shown in Table 1 which reflects the addition of the PowerPC MPC8540 and removal of the Intel Xscale processor per the discussions on February 5th. All DSPs have been mapped and documented. Validation will commence after component mappings are complete.

## Data Sources

To gather the data for he following are links to the data sources referenced in the DSP mappings. The data sheets are typically sufficient to identify the processor’s precision.

Table 1: Hyperlinks to data Files used in DSP file generation process

| DSP | Datasheet | Instruction Set | Library | Power |
| --- | --- | --- | --- | --- |
| TMS320C6205-200 | SPRS106G | SPRU731 SPRU198i | spru402b online benchmarks | SPRA486c |
| TMS320C6713B-300 | SPRS294B | SPRU733 SPRU198i | spru657b online benchmarks | SPRA889 |
| TMS320C6455-850 | SPRS276H | SPRU732f SPRU198i | sprueb8a SPRA750d online benchmarks SPRU972 (VCP2) SPRU973 (TCP2) | SPRAAE8B |
| TMS320C6416T-850 | SPRS226J | SPRU732f SPRU198i | sprueb8a SPRA750d (VCP) SPRA749b (TCP) | SPRAA45 |
| TMS320VC5502-300 | SPRS166J | SPRU374g | online benchmarks spru422i | SPRA993 |
| TMS320VC5416-160 | SPRS095O | SPRU131g SPRU172c | online benchmarks spru518d | SDR 05 Paper |
| ADSP-BF532 | ADSP-BF532 | Blackfin_PRM Instruction Set | BF Code Examples | Power |
| ADSP-21362 | ADSP-21362 | ADSP_2136x_PGR | ADSP2136 Examples | Power |
| ADSP-TS201S | ADSP-TS201S | Programmer’s Reference | TS201 Examples | Power |
| ADSP-2191M | ADSP2191M | Instruction Set Reference | Examples | Power |
| Pentium IV 661 | 30638203 31030802 | 253666 253667 |  | 86 W (TDP) |
| MPC8540 | MPC8540 | MPC8540RM E500Core RM Book E RM EREF | Libraries | MPC8540 |
| DSP56321 | DSP56321 | EFCOP Instruction Set | Benchmarks (56300 Family Reference, App B) |  |
| DSP56F8367 | MC56F8367 | DSP56800ERM | Code library |  |
| MSC8101 | MSC8101 | MSC8101RM (reg) SC140 RM | ??? | MSC8101 |
| MSC7116 | MSC7116 | MSC7116RM SC140 RM | ??? | Power Calculation |
| ARM 968E-S | Technical Reference Manual DDI0240A | DDI0311 ARM Architecture Reference Manual | ??? | ARM 968E-S |
| ARM1020E | Technical Reference Manual | DDI0177 ARM Architecture Reference Manual | ??? | ARM1020E |
| ARM1136J(F)-S | Technical Reference Manual | ARM Architecture Reference Manual Floating Point Coprocessor | ??? | ARM1136J(F)-S |
| ARM1136J-S | Technical Reference Manual | ARM Architecture Reference Manual Floating Point Coprocessor | ??? | ARM1136J-S |

| Manufacturer | Website |
| --- | --- |
| Texas Instruments | www.ti.com |
| Analog Devices | www.analog.com |
| Intel | www.intel.com |
| Freescale | www.freescale.com |
| ARM | www.arm.com |

## Characteristics

Characteristics are intended to model features of a DSP that when present will so dramatically alter the code that the typical modifier equations is not appropriate. In mappings, if the component lists a requirement which the DSP does not have as a characteristic, then the mapping will fail. For example if a component assumes that the support for floating point operations, the engine will not attempt to map the component onto a DSP that can only implement fixed point operations. Likewise for a component which assumes fixed point operation, the engine will not attempt to map the component onto a DSP that supports only floating point operations (as it may not have sufficient precision). If the distinction does not matter to the implementation, then the component file should not list the DSP characteristic as a requirement.

Table 2 provides a listing of DSP characteristics which were identified and used in Release 1.1. In general, these should be identified from supported addressing modes and data formats. The exceptions to this rule are the following:

- indexed addressing whose effect is simple enough that it can be modeled as a modifier

- direct and indirect addressing which every processor supports

- pre- and post-fix increment/decrementing which Release 1.1 is modeling every DSP as supporting.

Table 2: Characteristics identified and used in Release 1.1

| Characteristic | Description |
| --- | --- |
| Fixed | Supports fixed point processing |
| Round | Processor includes instructions for single-cycle rounding of results in various Q notations. |
| Float | Supports floating point processing |
| Double | Supports double floating point processing |
| Circular | Supports circular addressing modes |
| Reverse | Supports bit-reverse addressing |
| RISC | DSP is a RISC processor. Note that the cycle estimation process is not as robust for RISC processors because of the variable time to implement instructions and the high probability of stalls due to inter-instruction contention for resources and data dependencies. |
| Saturate | Supports saturation arithmetic (eliminates overflow condition) |
| Scaling | Implements rounding with scaling (shift left, shift right) |

## Instruction Modifiers

Modifiers are intended to model specialized hardware within a DSP that permits multiple operations to be executed in a single cycle, e.g., a MAC. For DSPs that have the appropriate hardware, instructions corresponding to the modifier can be eliminated from the pseudo-assembly code (e.g., accumulation from the MAC). To identify what modifiers a DSP has, its instruction set should be reviewed to identify which instructions it has that permit multiple operations to be executed in a single-cycle instruction. More complex specialized hardware which require numerous cycles to complete (e.g., a filter co-processor or a Viterbi co-processor) should not be modeled as an instruction modifier. Instead, these should be incorporated into the associated library code for a component. Note that some DSPs have specialized modifiers that would require a significant rewrite of the basic pseudo-code to appropriate leverage the modifiers. To model this situation, the component should treat the DSP as having a specialized library for that component.

Identified instruction modifiers for Release 1.1 are summarized in Table 3. Note that some modifiers are catch-alls for a collection of instruction modifiers that would be too tedious to model individually. For example, EXTRACT models various instructions that support various combinations of typical bit manipulation operations (e..g, shifting, bit extraction and insertion, and masking).

Table 3: Identified instruction modifiers in Release 1.1

| Instruction Modifier | Operation | Modeled Effect |
| --- | --- | --- |
| ABSALU | A typical ALU instruction (ADD, SUB) is combined with an absolute value operation. Useful for distance calculations and L1 norms. | Cycles associated with subsequent abs eliminated |
| ADDSUB | The ALU can simultaneously add and subtract two numbers and/or add/subtract two numbers of the native precision | Consecutive adds/subtracts eliminated |
| AVG | The processor implements (A+B)/2 | Eliminates a left shift when following an add |
| BDEC | The processor decrements a counter and branches | Counter decrement cycles eliminated in loops |
| BPOS | The processor branches on general conditions, e.g., R>0 | Cycle eliminated for generating branch condition |
| BITR | The processor reverses the bits of a register in a single cycle | Cycles for process to bit-reverse an indexed array are eliminated. 1 cycle per array element added back in. |
| COND_EXEC | All instructions are executed conditionally | Cycles consumed for short control branches are eliminated |
| COND_MOV | All memory/move operations can be executed conditionally | Cycles consumed for short control branches related to memory are eliminated |
| CPX_MPY | A single cycle complex multiplication. Note that several DSPs have an instruction which implements a complex multiplication (or LMS update or an FIR cycle) but these are multi-cycle instructions. Here, we’re only modeling situations where x_r * y_r – x_i*y_i, x_i*y_r + x_r*y_i. So far, this has also included xyH. | Complex multiplication cycles (6) reduced to 1 per complex multiplication. |
| EXTRACT | The processor is capable of detailed bit manipulation in a single cycle. This takes various forms in different instructions, but a minimum requirement is the ability to extract out a specified set of bits from a word and then pack them into bytes | Bit manipulation cycles cut in half. |
| GMPY | The processor supports Galois Field arithmetic (useful in some error correction codes). | Cycles required to mimic Galois Field arithmetic are eliminated with one cycle per Galois Field arithmetic operation added in. |
| INDEX | The processor supports indexed addressing, e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2. | Cycles used to separately add together register with an address register eliminated. |
| MAC | A single-instruction (functional unit) multiplication and accumulation. Note that when both a multiplier and an ALU are required to implement this, a processor is not considered to exhibit the MAC modifier unless it does not support VLIW | Accumulate cycles eliminated |
| MAX | The processor does the following (and the reverse for a MIN): if A>B, A-> dst | Cycles required to perform move following comparison operation eliminated |
| MAX2 | The process performs the MAX operation for two pairs of words of native precision. | Cycles required to perform both moves and one comparison eliminated |
| MEM2 | The data bus width of a processor is such that a single instruction fetches 2 words. | Memory cycles cut in half (rounded up). |
| MEM4 | The data bus width of a processor is such that a single instruction fetches 4 words. | Memory cycles cut in fourth (rounded up). |
| NOREG | The processor memory maps all registers so there’s no need for an instruction to load registers from memory. Processors which do this, however, tend to clock much slower | All cycles used to move |
| SAD | Sum of absolute differences. | Absolute values removed. A special case of ABSALU. |
| SIDE_SUM | Adds up bytes in a word. | Eliminate cycles of sum of byte-packed words |
| VECT | A single cycle complex multiplication. Note that several DSPs have an instruction which implements a complex multiplication (or LMS update or an FIR cycle) but these are multi-cycle instructions. Here, we’re only modeling situations where x_1 * y_1 + x_2*y_2, x_3*y_3 + x_4*y_4. | Complex multiplication cycles (6) reduced to 1 per complex multiplication. |
| VSL | A process by which a register is shifted left and an input 1 or 0 is appended to the right most bit. Useful for keeping track of paths (saves an instruction). | Cycle saved per path update in |
| ZOL | The processor supports a form of zero-overhead (hardware) looping wherein loop instructions are placed in a hardware buffer and repeated a specified number of times. | All loop control cycles eliminated (branch, compare, decrement). One cycle added to set loop counter. |

## VLIW

Very-Long-Instruction-Word (VLIW) is how super-scalar DSP architectures are implemented. In effect in a single VLIW fetch, multiple instructions are returned and dispatched. This is accomplished by having multiple functional units on a processor which execute the instructions. For example a DSP that has both a multiplier and an ALU could support simultaneous execution of instructions for both the ALU and the multiplier. In general the number of instructions that can be simultaneously executed will be limited by the number of functional units or the program bus width. For example, a DSP might have two multipliers and two ALUs, but only be capable of executing two instructions at a time.

So to model a DSP that supports VLIW, the following steps should be taken. First, it should be determined if the DSP has multiple functional units. These functional units should be classified as memory units, ALU units, and multiplier units. For our purposes, a shifter is considered an ALU unit (it’s not, but situations where this assumption doesn’t work should be rare). Note that DSP architectures vary significantly, so some discretion will be needed when making these classifications.

Then the maximum number of instructions that can be executed should be determined as well as the number of instructions of the each function unit classification (Multiplier, ALU, memory). On a RISC processor, the maximum number of executable instructions is equal to the maximum number of instructions that can be retired per cycle.

Note that some DSPs have wider than normal data-memory paths that allow a greater bandwidth when moving data between processing registers and memory. However, being able to fetch 2 or 4 or 8 words with a single instruction should not be modeled as VLIW as distinct instructions are not being used in this process. The key phenomenon being modeled is the capacity to simultaneously execute multiple distinct instructions.

## SIMD Estimation

When a DSP supports Single-Instruction-Multiple-Data instructions, the same instruction can be applied to multiple data elements in a single cycle. Generally, this is realized by having functional units that can operate with word-widths smaller than the native word width. For example a DSP in the TI 6455 family natively works with 32-bit words, but its ALUs can also simultaneously add 4-pairs of 8-bit words. However, there are processors that include secondary functional units that execute the same instructions as the primary functional units when an SIMD bit is set.

Because we are not creating a detailed computational model for each DSP (which would be almost equivalent to creating a compiler for each DSP), SIMD has to be applied somewhat coarsely. Thus we only treat a DSP as a SIMD processor if it supports SIMD operations in its multipliers, ALUs, and memory units (typically only the multipliers or ALUs will be limiting). Note that some DSPs have wider than normal data-memory paths that allow a greater bandwidth when moving data between processing registers and memory. In this situation, rather than (or in addition to) modeling the DSP as SIMD, the DSP is modeled as having instruction modifiers MEM2, MEM4, or MEMx to model the double, quadruple, or x-times memory fetched with a single instruction.

## Power Calculation

In this implementation, the engine estimates energy consumption by multiplying the peak power draw (peak core power + typical I/O values) by the estimated processing time. This is inline with an assumption that components are implemented for rapid execution which entails maximum utilization of chip resources. I/O power should be left at typical values when the data is available.

The kinds and quality of documentation available for estimating peak power draw vary significantly from processor to processor so various methods are needed to estimate power consumption. The following are some of the more typical methods.

### Spreadsheet Estimation

Change the CPU utilization rate to 100% while leaving the other values at the levels initially set by the manufacturer (for peak core power + typical I/O). While not seen in this survey, there may be processor spreadsheets that ask for estimations of bit toggling rates (a common practice in FPGA data sheets). Bit toggling rates should generally not be set above 50% (meaning that half the time bits flip).

### Extrapolation

Some vendors will give measured power consumptions values with different levels of CPU activity/utilizations (e.g., 1 W at 25% and 2.5 W at 75%). A reasonable first-order approximation to estimating peak power consumption in this situation is to perform a linear extrapolation to 100% utilization. To make this extrapolation, you should be cognizant that power consumption is the sum of dynamic power consumption (power consumed while transistors are actively switching) and static power consumption (power consumed due to leakage). Sometimes a static power consumption value is given and sometimes not. If it is not given, then you need to solve the implied linear system of equations using at least to different activity levels.

For example the following set of equations would yield peak_dyn = 3 W, static = 0.25 W for a maximum power level of 3.25 W.

1 = peak_dyn * 0.25 + static

2.5 = peak_dyn *0.75 + static

### Current x Voltage

Sometimes the only documentation available gives peak current draws and supply voltages for different parts of the chip. In such a situation peak power consumption is simply the sum of the products of peak current draws and supply voltages. In some situations, it may be necessary to extrapolate current draws for chip components as we did with extrapolation of peak power consumption.

### Scaling from a Reference DSP

Sometimes power estimates are known for one DSP in a family, but not another. When the same fabrication technology is used for both DSPs (e.g., both are 0.13 m processes).  In this case, you should leverage the fact that P  Cv2f where C is capacitance, v is voltage, f is frequency, and P is power consumption. By assuming that C and the proportionality factor remains the same for the same DSP family with the same fabrication technology, the unknown power estimation can be calculated from the known DSP as

Punknown = Pknown x (vunknown / vknown)2 x (funknown / fknown)

When fabrication technologies change (but the chip architecture remains the same), the methods used in [2] should be applied.

Note that this approach should not be used if the chip architectures are different.

## References

[1]

[2] J. Neel, S. Srikanteswara, J. Reed, P. Athanas, “A Comparative Study of the Suitability of a Custom Computing Machine and a VLIW DSP for Use in 3G Applications,” IEEE Workshop on Signal Processing Systems SiPS2004, Oct 13-15, 2004, pp. 188-193.

# TMS320C6205-200

The following provides details on how parameters for the data file for the TMS3206205-200 DSP were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS3206205-200

## Information from DataSheet

Native Precision: 16 (multiplication limiting)

Instruction Bits: 32

Clock Rate (MHz): 200

## Peak Power Calculation

[] gives two estimates of power consumption, one at 75% “high” / 25% “low”, and one at 50% “high” / 50% “low”. Forming a linear extrapolation as suggested in the document, from 50% high at 1.15 W to 75% high at 1.32 W yields an estimate of 2 x (1.32-1.15) + 1.15 = 1.49W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Saturate

Circular

Indexed

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction (excepting saturation and sign manipulation).

BPOS

Positive register values can be used for registers (not an actual BPOS instruction, but a conditional branch can be made equivalently)

Cond_Exec

All instructions on the TMS3206205 can be executed conditionally.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

VLIW is at the heart of the TMS3206205-200 architecture. According to [], it supports up to 8 simultaneous instructions executed over 6 ALUs and two multipliers. Two of these ALUs, however, are primarily used to support memory loading/storing operations and as such are scored as memory units instead. The assigned parameters are summarized as follows.

VLIW_flag: 1

VLIW_max: 8

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

According to [], the TMS3206205-200 has two instructions which effect SIMD-like operation (ADD2, SUB2). However, broader support is not available (e.g., a MPY2) so SIMD is flagged as 0 for this processor.

## Benchmarked Routines []

LMS FIR Update (even)

Autocorrelation (factor of 8)

Complex Bit-Reverse

Complex Forward FFT (radix-2)

Complex Forward FFT (radix-4)

Mixed radix FFT (scaled, rounded, 16 bit)

Complex FIR Filter

FIR Filter

FIR Filter (radix 4)

FIR Filter (radix 8)

FIR Filter (symmetric)

IIR Filter (5 Coefficients)

IIR Filter (all pole lattice)

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

## References

[] TMS320C6205 Fixed Point Digital Signal Processor, SPRS106G, July 2006

[] Kyle Castille, TMS320C62x/C67x Power Consumption Summary, SPRA486c, July 2002. Available online: http://focus.ti.com/dsp/docs/dspsupporttechdocsc.tsp?sectionId=3&tabId=409&familyId=326&abstractName=spra486c

[] TMS320C62x DSP CPU and Instruction Set Reference Guide, SPRU731, July 2006.

[] TMS320C62x DSP Library Programmer’s Reference, SPRU402B, October 2003.

# TMS320C6713B-300

The following provides details on how parameters for the data file for the TMS320C6713B-300 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS320C6713B-300

## Information from DataSheet []

Native Precision: 32

Instruction Bits: 32

Clock Rate (MHz): 300

## Peak Power Calculation

[] provides a spreadsheet for calculating power consumption. At 100% CPU activity and the remaining parameters left at their defaults, the spreadsheet gives a total power consumption of 1.76 W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Float

Double

Saturate

Circular

Indexed

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction (excepting saturation and sign manipulation).

BPOS

Positive register values can be used for registers (not an actual BPOS instruction, but a conditional branch can be made equivalently)

Cond_Exec

All instructions on the TMS320C6713B-300 can be executed conditionally.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

VLIW is at the heart of the TMS320C6713B-300 architecture. According to [], it supports up to 8 simultaneous instructions executed over 6 ALUs and two multipliers. Two of these ALUs, however, are primarily used to support memory loading/storing operations and as such are scored as memory units instead. The assigned parameters are summarized as follows.

VLIW_flag: 1

VLIW_max: 8

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

According to [], the TMS320C6713B-300 has two instructions which effect SIMD-like operation (ADD2, SUB2). However, broader support is not available (e.g., a MPY2) so SIMD is flagged as 0 for this processor.

## Benchmarked Routines []

(16-bit fixed, single and double precision for all of the following)

LMS FIR Update (even)

Autocorrelation (factor of 8)

Complex Bit-Reverse

Complex Forward FFT (radix-2)

Complex Forward FFT (radix-4)

Mixed radix FFT (scaled, rounded, 16 bit)

Complex FIR Filter

FIR Filter

FIR Filter (radix 4)

FIR Filter (radix 8)

FIR Filter (symmetric)

IIR Filter (5 Coefficients)

IIR Filter (all pole lattice)

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

## References

[] TMS320C6713B Floating Point Digital Signal Processor, SPRS294B, June 2006

[] Ivan Garcia, TMS320C6711D, C6712D, C6713B Power Consumption

Summary, SPRA889c, August 2005.

[] TMS320C67x/C67x+ DSP CPU and Instruction Set Reference Guide, SPRU733a, November 2006

[] TMS320C67x DSP Library Programmer’s Reference, SPRU657B, March 2006.

# TMS320C6455-850

The following provides details on how parameters for the data file for the TMS320C6455-850 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS320C6455-850

## Information from DataSheet []

Native Precision: 32

Instruction Bits: 32

Clock Rate (MHz): 850

The TMS320C6455-850 is a 64x+ architecture (some instructional differences between 64xx and 64x+ architectures)

## Peak Power Calculation

[] provides a spreadsheet for calculating power consumption. At 100% CPU activity and the remaining parameters left at their defaults, the spreadsheet gives a total power consumption of 2.20 W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Saturate

Circular

Indexed

Reverse

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ADDSUB

Using its ALU units, the TMS320C6455-850 supports an operation whereby both the addition and subtraction of two numbers are simultaneously calculated.

AVG

Using the multiplier units, the TMS320C6455-850 supports an averaging operation which is equivalent to an addition and a right shift.

BDEC

Using its ALU units, the TMS320C6455-850 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison) and decrements the register. This effectively collapses up to three instructions into 1.

BITR

Reverses the bits of a register in a single instruction.

BPOS

Using its ALU units, the TMS320C6455-850 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

Cond_Exec

All instructions on the TMS320C6455 can be executed conditionally.

EXTRACT

Using its multiplier units, the TMS320C6455-850 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

GMPY

The 6455 supports the calculation of Galois Field multiplications dual 16x16 (and 8x8) MACs on their .M (multiplier) units.

MAX

Using its ALU units, the TMS320C6455-850 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

ZOL

According to [], the TMS320C6455-850 implements what they call SPLOOP instruction (Software Pipelined Loop Buffer) wherein looped code is stored in a buffer and repeated. This eliminates the need for using a conditional instruction to control looping with each iteration and reduces number of program memory fetches (reduces power). For the purposes of this project, the TMS320C6455-850’s SPLOOP capability will be treated like a zero-overhead-loop instruction.

VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

VLIW is at the heart of the TMS320C6455-850 architecture. According to [], it supports up to 8 simultaneous instructions executed over 6 ALUs and two multipliers. Two of these ALUs, however, are primarily used to support memory loading/storing operations and as such are scored as memory units instead. The assigned parameters are summarized as follows.

VLIW_flag: 1

VLIW_max: 8

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

According to [] the TMS320C6455-850 supports a wide variety of SIMD instructions with a minimum word size of 8.

## Benchmarked Routines []

(16-bit fixed, single and double precision for all of the following)

LMS FIR Update (even)

Autocorrelation (factor of 8)

Various FIR/IIR Filters

Various FFT Implementations

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

Additionally, the TMS320C6455-850 has Viterbi and Turbo-Code co-processors which are treated as benchmarked routines for the purposes of this project.

## References

[] TMS320C6455 Fixed Point Digital Signal Processor, SPRS276H, October 2007

[] Gustavo Martinzez, TMS320C6455, C6455 Power Consumption Summary

Summary, SPRAAE8B, October 2007.

[] TMS320C64x/C64x+ DSP CPU and Instruction Set Reference Guide, SPRU732e, November 2007

[] TMS320C64x+ DSP Little-Endian DSP Library Programmer’s Reference, SPRUE8A, September 2007.

# TMS320C6416T-850

The following provides details on how parameters for the data file for the TMS320C6416T-850 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS320C6416T-850

## Information from DataSheet []

Native Precision: 32

Instruction Bits: 32

Clock Rate (MHz): 850

The TMS320C6416T-850 is a 64xx architecture (some instructional differences between 64xx and 64x+ architectures)

## Peak Power Calculation

[] provides a spreadsheet for calculating power consumption. At 100% CPU activity and the remaining parameters left at their defaults, the spreadsheet gives a total power consumption of 1.74 W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Indexed

Saturate

Circular

Reverse

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

(NO) ADDSUB

Unlike the C6455, the TMS320C6416T-850 does not implement ADDSUB.

AVG

Using the multiplier units, the TMS320C6416T-850 supports an averaging operation which is equivalent to an addition and a right shift.

BDEC

Using its ALU units, the TMS320C6416T-850 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison) and decrements the register. This effectively collapses up to three instructions into 1.

BITR

Reverses the bits of a register in a single instruction.

BPOS

Using its ALU units, the TMS320C6416T-850 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

Cond_Exec

All instructions on the TMS320C6416T can be executed conditionally.

EXTRACT

Using its multiplier units, the TMS320C6416T-850 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

(NO) GMPY

Unlike the C6455, the TMS320C6416T-850 does not implement Galois Field Multiplications.

MAX/MIN

Using its ALU units, the TMS320C6416T-850 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

(NO) ZOL

Unlike the C6455, the TMS320C6416T-850 does not implement SPLOOP.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

VLIW is at the heart of the TMS320C6416T-850 architecture. According to [], it supports up to 8 simultaneous instructions executed over 6 ALUs and two multipliers. Two of these ALUs, however, are primarily used to support memory loading/storing operations and as such are scored as memory units instead. The assigned parameters are summarized as follows.

VLIW_flag: 1

VLIW_max: 8

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

According to [] the TMS320C6416T-850 supports a wide variety of SIMD instructions with a minimum word size of 8.

## Benchmarked Routines []

LMS FIR Update (even)

Autocorrelation (factor of 8)

Various FIR/IIR Filters

Various FFT Implementations

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

Additionally, the TMS320C6416T-850 has Viterbi and Turbo-Code co-processors which are treated as benchmarked routines for the purposes of this project.

## References

[] TMS320C6416T Fixed Point Digital Signal Processor, SPRS226J, July 2006

[] Todd Hiers and Matthew Webster, TMS320C6414T/15T/16T Power Consumption Summary, SPRAA45, August 2004.

[] TMS320C64x/C64x+ DSP CPU and Instruction Set Reference Guide, SPRU732e, November 2007

[] TMS320C64x DSP Little-Endian DSP Library Programmer’s Reference, SPRU565b, October 2003.

# TMS320VC5502-300

The following provides details on how parameters for the data file for the TMS320VC5502-300 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS320VC5502-300

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz): 300

The C55 provides two 17x17 bit MAC units, a 40-bit ALU, a barrel shifter a 16-bit ALU, four accumulators and units for data and program address generation. It supports all C54 instructions including block repeats and the various addressing modes, including the C54 parallel operations (e.g., FIRS, LMS). Technically, the C55 is not a traditional VLIW (superscalar) architecture as arbitrary combinations of instructions cannot be issued. However, it does enough instructional parallelism (e.g., MAC::ADD) that we will consider it capable of implementing a multiplication and an ALU instruction simultaneously (maximum of two operations).

Because of its data memory management unit and architectural implementation, all instructions can access data memory and manipulate data address registers and local registers are generally not used for computation (the exception being two accumulators and a temporary register). Because of this, cycles are not consumed in the reading and writing and are instead embedded in most instructions (though data addresses can be separately modified as needed). Note that in practice, this has the effect of slowing down the overall clock rate as more functions need to be accomplished in a single cycle. This behavior will be modeled as a synergistic cycle modifier (with itself) to avoid miscalculations in data memory requirements that eliminates all data memory cycles (e.g., target – target).

Note that unlike the TI 6000 series processors, only branching-type instructions are executed conditionally.

## Peak Power Calculation

[] provides a spreadsheet for calculating power consumption. At 100% CPU activity and the remaining parameters left at their defaults, the spreadsheet gives a total power consumption of 488 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Saturate

Circular

Reverse

Indexed

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ABDST

Accumulates the absolute distance between two vectors, X, Y. Specifically, subtracts Y from X, evaluates the absolute value and accumulates the result with a stored result. However, this “instruction” uses two units and as such is already modeled in the VLIW estimate.

ADDSUB

Using its 40-bit ALU, the TMS320VC5502 supports an operation whereby both the addition and/or subtraction of two numbers are simultaneously calculated. As this is the only SIMD-like operation on the 5416, this is modeled as a modifier.

BPOS

Using its ALU units, the TMS320VC5502 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

EXTRACT

The TMS320VC5502 supports several operations (e.g., BFXPA, BFXTR) which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

MAC

A single-cycle multiply and accumulate

NOREG

Because it memory maps all registers, there’s no need to use instructions to move data from memory to the inputs of functional units. Note that moving operands between internal registers (e.g., loading a counter or shifting data between accumulators) still requires the use of a move operator.

ZOL

According to [], the TMS320VC5502 implements a block-repeat operation.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

VLIW is not exactly natural to the TMS320VC5502-300 architecture, but it does support some computational parallelism by virtue of having three data paths, two MAC units and an ALU. However, only two instructions can be executed at a time. This will be modeled as follows.

VLIW_flag: 1

VLIW_max: 2

VLIW Memory: 2 (as it relates to move instructions)

VLIW ALU: 1

VLIW Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

According to [] the TMS320VC5502-300 does not support SIMD.

## Benchmarked Routines []

(16-bit fixed, single and double precision for all of the following)

LMS FIR Update (even)

Autocorrelation (factor of 8)

Various FIR/IIR Filters

Various FFT Implementations

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

Additionally, the TMS320VC5502-300 has Viterbi and Turbo-Code co-processors which are treated as benchmarked routines for the purposes of this project.

## References

[] TMS320VC5502 Fixed Point Digital Signal Processor Data Manual, SPRS166J, August 2006.

[] Gustavo Martinzez, TMS320VC5501/02 Power Consumption Summary, SPRA993, January 2007.

[3] TMS320C55x DSP Mnemonic Instruction Set Reference Guide, SPRU 374g, October 2002.

[5] TMS320C55x Technical Overview SPRU 393, February, 2000.

# TMS320VC5416-160

The following provides details on how parameters for the data file for the TMS320VC5416-160 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: TMS320VC5416-160

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz): 160

The C54 provides a 17x17 bit multiplier, a 40-bit ALU, a barrel shifter, a 40-bit adder, a compare select and store unit (CSSU) and units for data and program address generation. Despite its large number of computational units, the C54 only implements a single instruction at a time (in general). However, a handful of instructions (e.g., FIRS which implements a symmetric FIR and LMS which implements the updating process of an LMS filter) can make use of multiple computational units simultaneously. Rather than considering this limited subset of capabilities as constituting a VLIW processor, these special cases should be captured in relevant library code.

Because of its data memory management unit and architectural implementation, all instructions can access data memory and manipulate data address registers and local registers are generally not used for computation (the exception being two accumulators and a temporary register). Because of this, cycles are not consumed in the reading and writing and are instead embedded in most instructions (though data addresses can be separately modified as needed). Note that in practice, this has the effect of slowing down the overall clock rate as more functions need to be accomplished in a single cycle. This behavior will be modeled as a synergistic cycle modifier (with itself) to avoid miscalculations in data memory requirements that eliminates all data memory cycles (e.g., target – target).

## Peak Power Calculation

[3] estimates typical power consumption for the TMS320VC5416 at 160 mW. (No spreadsheets were available from TI for this processor.) Assuming the same static/dynamic power ratio seen in other processors, this implies a peak power rating of 270 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Saturation arithmetic

Circular addressing

Bit-reversed addressing (for FFTs)

Indexed addressing (e.g., *R1+R2 where value in R1 is loaded and then R1 is offset by R2)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ABDST

Accumulates the absolute distance between two vectors, X, Y. Specifically, subtracts Y from X, evaluates the absolute value and accumulates the result with a stored result. However, this “instruction” uses two units and as such is already modeled in the VLIW estimate.

ADDSUB

Using its 40-bit ALU, the TMS320VC5416-160 supports an operation whereby both the addition and/or subtraction of two numbers are simultaneously calculated. As this is the only SIMD-like operation on the 5416, this is modeled as a modifier.

BPOS

Using its ALU units, the TMS320VC5416-160 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

MAX/MIN

Using its ALU units, the TMS320C5416-160 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

MAC

A single-cycle multiply and accumulate

NOREG

Because it memory maps all registers, there’s no need to use instructions to move data from memory to the inputs of functional units. Note that moving operands between internal registers (e.g., loading a counter or shifting data between accumulators) still requires the use of a move operator.

ZOL

According to [], the TMS320C5416-160 implements a block-repeat operation.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

For the purposes of this project, VLIW will not be considered part of the TMS320VC5416-160 architecture although a handful of instructions can make use of the multiple computational units.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

For the purposes of this project, the TMS320VC5416-160 architecture will not be considered to implement SIMD. There is some SIMD like capabilities supported by the accumulators, but this is intended to be reflected in the ADDSUB modifier.

## Benchmarked Routines []

(16-bit fixed, single and double precision for all of the following)

LMS FIR Update (even)

Autocorrelation (factor of 8)

Various FIR/IIR Filters

Various FFT Implementations

Vector Dot Product

Vector Dot Product (and Square)

Maximum Value of a Vector

Index of Maximum Element of Vector

Minimum Value of Vector

32-bit Vector Multiply

32-bit Vector Negate

16-bit Reciprocal

Sum of Squares

Weighted Vector Sum

Matrix Multiplication

Matrix Transpose

Max Exponent of a Vector

Move Block of Memory

Q15 to Float Conversion

Float to Q15 Conversion

Additionally, the TMS320VC5416-160 has Viterbi and Turbo-Code co-processors which are treated as benchmarked routines for the purposes of this project.

## References

[] TMS320VC5416 Fixed Point Digital Signal Processor Data Manual, SPRS095O, January 2005.

[2] TMS320C54x DSP Reference Set Volume 1: CPU and Peripherals, SPRU131G, March 2001.

[3] R. Chembil, J. Kim, J. Lee, D. Ha, C. Patterson, J. Reed, “Reconfigurable Modem Architecture for CDMA Based 3G Handsets,” SDR Forum 2005.

[4] TMS320C54x DSP Reference Set Volume 2: Mnemonic Instruction Set, SPRU172C, March 2001.

[5] TMS320C54x DSP Library Programmer’s Reference, SPRU518d October 2004.

# ADSP-BF532

The following provides details on how parameters for the data file for the ADSP-BF532 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ADSP-BF532

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz): 400 (only clock rate supported by BF532)

The BF532 provides two 16-bit multipliers, two 40-bit accumulators, two 40-bit ALUs, four video ALUs, and a 40-bit shifter. It supports SIMD and a version of instruction parallelism. While it supports a data address generation unit – generating these addresses still consume cycles.  Supports some conditional execution, but not in general.

## Peak Power Calculation

[3] gives a typical dynamic internal power drain of 159 mA for a voltage of 1.2 V and frequency of 400 MHz (the configuration assumed in this document). That corresponds to a dynamic internal power consumption of 191 mW. Static current draw is estimated at 130 mA for a static power consumption of 156 mW for a total estimate of 347 mW. Typical external power consumption is then estimated at 171 mW.

Peak power is then 347 mW (internal) + 171 mW (external) = 518 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Rounding

Saturation arithmetic

Circular addressing

Bit reversed addressing (for FFTs)

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ADDSUB

Using its ALU units, the ADSP-BF532 supports an operation whereby both the addition and/or subtraction of two numbers are simultaneously calculated. This is in addition to other SIMD operations.

BPOS

The ADSP-BF532 supports conditional execution of its branches, calls, and jumps based on status in the CC register.

EXTRACT

The ADSP-BF532 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

MAX/MIN

Using its ALU units, the ADSP-BF532 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle. This is implemented in the compare select unit.

MAC

A single-cycle multiply and accumulate

ZOL

According to [], the ADSP-BF532 implements zero-overhead looping.

## VLIW Information

Very Long Instruction Word (VLIW) is a characteristic of a superscalar architecture wherein multiple instructions can be simultaneously executed across several functional units.

Technically, the BF532 is not considered to be a superscalar architecture. However, it can formally execute a 32-bit instruction with two 16-bit instructions. In practice, this generally means 2 arithmetic instructions and 2 data memory management instructions. Accordingly, this will be treated as having VLIW with:

2 Multipliers

2 ALUs

2 Memory

with a maximum of 4 instructions executed per cycle. Technically, this will create somewhat misleading results as it is not possible to execute 2 multiplier instructions with 2 ALU instructions, but it is rare (i.e., I don’t know when) where 2 Multiplier/ALU instructions are not accompanied by two Memory loads to support the multiplier/ALU processes.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

For the purposes of this project, the ADSP-BF532 architecture will be considered to implement SIMD. We will treat this as native 16-bit operation with minimum 8-bit word size. In practice some 32-bit operations can be implemented, but they preclude the parallelism of VLIW so this should “even out”.

## Benchmarked Routines []

FFTs

Convolutional Encoders

Interpolation/Decimation Filters

Digital Modulation

## References

[] ADSP-BF531/ADSP-BF532/ADSP-BF533, Rev e, July 2007.

[2] ADSP-BF53x/BF56x Blackfin® Processor Programming Reference, Rev 1.2, February 2007.

[3] Joe B., “Estimating Power for ADSP-BF531/BF532/BF533 Blackfin® Processors,” EE-229, Rev 2 September 7, 2006

[4] http://www.analog.com/processors/blackfin/technicalLibrary/manuals/codeExamples.html

# ADSP-21632

The following provides details on how parameters for the data file for the ADSP-21632 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ADSP-21632

## Information from DataSheet []

Native Precision: 64 (technically 32, but needed to fit into current SIMD model).

Clock Rate (MHz): 333 (only clock rate supported by ADSP-21632)

The ADSP-21632 consists of two processing elements, each of which comprises an ALU, multiplier (and accumulator), shifter, and data register file. These two processing elements permit a different version of SIMD where the same instructions are dispatched to both processing units when SIMD mode is enabled which means that one processing element (PEx) is always executing while the other processing element (PEy) only executes when PEy is enabled.

## Peak Power Calculation

For a clock rate of 333 MHz, [3] gives a peak dynamic internal current drain of 848 mA and a static current drain of 550 mA which corresponds to 1.68 W at a voltage of 1.2 V. [3] then gives an average external power consumption value of 99 mW. This gives a total power consumption of 2.67 W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Float

Rounding

Saturation arithmetic

Circular addressing

Bit reversed addressing (for FFTs)

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ADDSUB

Using its ALU units, the ADSP-21632 supports an operation whereby both the addition and/or subtraction of two numbers are simultaneously calculated. This is in addition to other SIMD operations.

AVG

Using its ALU units, the ADSP-21632 can add together two numbers and right shift by 1 realizing an averaging operation.

BPOS

The ADSP-21632 supports conditional execution of its branches, calls, and jumps based on status in the CC register.

Cond_Exec

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ADSP-21632 can be conditionally executed, so it is modeled as supporting conditional execution.

EXTRACT

Using its shifters, the ADSP-21632 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

MAX

Using its ALU units, the ADSP-21632 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

MAC

A single-cycle multiply and accumulate.

ZOL

According to [], the ADSP-21632 implements zero-overhead looping.

## VLIW Information

Each processing element can execute a multiplication instruction with an ALU instruction or two ALU instructions in parallel. Additionally, two data load/stores can be done in each instruction. This will be modeled as follows:

VLIW_max: 4

Memory: 2

ALU: 2

Mult: 1

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

The ADSP-21632 bills itself as an SIMD architecture, but this is slightly different than the way in which we’ve defined SIMD for this project. Specifically, the ADSP-21632 has two processing elements with their own registers that replicate the multiplier (MAC), ALU and shifter and execute the same instruction on both processing elements.

To model this, we’re treating the architecture as a native 64-bit architecture with a minimum word size of 32 bits.

## Benchmarked Routines []

Block FIR

Complex FIR

Complex Vector Add

Vector Complex Dot Product

Complex Radix 2 FFT

Complex Radix-4 FFT

Single Sample FIR

Vector Maximum

## References

[] ADSP-21362/ADSP-21363/ADSP-21364/ADSP-21365/ADSP-21366 SHARC Data Sheet, Rev C, September 2007.

[2] ADSP-2136x SHARC® Processor Programming Reference, Rev 1.0, March 2007.

[3] C. Coughlin, “Estimating Power for the ADSP-21362 SHARC® Processors,” EE-277, Rev 1 January 31, 2006

[4] http://www.analog.com/processors/platforms/sharcSoftwareModules.html

# ADSP-TS201S

The following provides details on how parameters for the data file for the ADSP-TS201S were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ADSP-TS201S

## Information from DataSheet []

Native Precision: 64 (32x32 bit multiplication is largest multiplication, but can do 4 16x16 for 8 different 16 bit input words)

Clock Rate (MHz): 600

The ADSP-TS201S consists of two processing elements called compute blocks which consist of an ALU, a MAC, a barrel shifter, and a communications logic unit which implements add-compare-select, dispreading, trellis, and TMAX operations. Additionally, it has two data address generation unit (called integer ALUs) whose quad data word permits each unit to move 128 bits (4 32 bit words) in a single cycle. Everything is effectively SIMD, and the ALU, MAC and shifter can operate on 64 bit operands (single cycle instructions). The MAC interestingly supports complex multiplications.

Up to four instructions can be executed per cycle, which can be executed conditionally. Combined with SIMD, this means that the ADSP-201S can support 8 16-bit MACs per cycle.

## Peak Power Calculation

Using the spreadsheet provided with [3] and keeping all defaults except changing CLU and Compute execution to high (to model peak power consumption) gives a total powe consumption of 3.72 W.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Float

Rounding

Saturation arithmetic

Circular addressing

Bit reversed addressing (for FFTs)

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

## Modifiers:

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ABSALU

Executes a typical ALU operation (add, subtract) and then evaluates absolute value of the result in a single cycle.

ADDSUB

Using its ALU units, the ADSP-TS201S supports an operation whereby both the addition and/or subtraction of two numbers are simultaneously calculated. This is in addition to other SIMD operations.

AVG

Using its ALU units, the ADSP-TS201S can add together two numbers and right shift by 1 realizing an averaging operation.

BPOS

The ADSP-TS201S supports conditional execution of its branches, calls, and jumps based on status in the CC register.

COND_EXEC

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ADSP-TS201S can be conditionally executed, so it is modeled as supporting conditional execution.

CPX_MPY

Implements a single cycle complex multiplication. Can save several cycles.

EXTRACT

Using its shifters, the ADSP-TS201S supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

MAX

Using its ALU units, the ADSP-TS201S supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle. This is implemented in the compare select unit.

MAC

A single-cycle multiply and accumulate.

SIDE_SUM

Adds up bytes in a word (can save accumulation cycles).

MEM2

Because of its 128 bit bus, only half as many cycles need to be consumed in memory operations as would be expected from the “native” 64 bit size.

ZOL

According to [], the ADSP-TS201S implements zero-overhead looping. In fact it supports two zero-overhead loops which would be useful in nested loops if we were modeling that.

## VLIW Information

The ADSP-TS201S can execute up to 4 instructions at a time with no more than 2 per CU and no more than 1 per CU element. There is actually a lengthy list of restrictions of which instructions can be done in parallel with others, particularly in terms of outputs, but we’re ignoring that. Additionally two data load/stores can be done in each instruction. However because the quad access (128 bits) is larger than the modeled native bit field (64 bits), we’re modeling this as a special memory modifier. Accordingly, VLIW processing will be modeled as follows:

VLIW_max: 4

Memory: 2

ALU: 4

Mult: 2

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

Typical word size of 64 bits

Minimum word size of 8 bits.

## Benchmarked Routines []

Floating point FIR, IIR

32-bit FFT (real, complex, float)

16-bit Fixed FFT (256,512)

LMS (float)

## References

[] ADSP-TS201 TigerSHARC® Processor Hardware Reference, Rev 1.1, December 2004.

[2] ADSP-TS201 TigerSHARC® Processor Programming Reference, Rev 1.1, April 2005.

[3] Greg F., “Estimating Power for ADSP-TS201S TigerSHARC® Processors,” EE-170, Rev 2 October 16, 2006 (Spreadsheet: http://www.analog.com/UploadedFiles/Associated_Docs/90718635EE170v02_4681b103_0341_452c_8799_912246926fe0.zip)

[4] http://www.analog.com/processors/tigersharc/technicalLibrary/codeExamples/codeExamples.html

# ADSP-2191M

The following provides details on how parameters for the data file for the ADSP-21632 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ADSP-2191M

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz): 160

The ADSP-2191M consists of a single processing element, which comprises an ALU, multiplier (and accumulator), shifter, and data register file, and two data address generation units. All processing is done in 16-bits, but conditional execution is supported

## Peak Power Calculation

Analog Devices did not have a power estimate for the ADSP-2191M available. However, [3] gives an estimate of power consumption of 525 mW at 50% internal loading. Assuming the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 426 mW

dynamic = 1.54 static

static = 241 mW

peak dynamic = 371 mW

This gives a total power consumption of 711 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Rounding

Saturation arithmetic

Circular addressing

Bit reversed addressing (for FFTs)

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The ADSP-2191M supports conditional execution of its branches, calls, and jumps based on status in the CC register.

COND_EXEC

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ADSP-2191M can be conditionally executed, so it is modeled as supporting conditional execution.

MAC

A single-cycle multiply and accumulate.

ZOL

Though it doesn’t call it as such, the ADSP-2191M implements zero-overhead looping (specifically it keeps track of begin and end of loops, manages a counter, and automates branching).

## VLIW Information

Each processing element can execute a multiplication instruction with an ALU instruction or two ALU instructions in parallel. Additionally, two data load/stores can be done in each instruction. This will be modeled as follows:

VLIW_max: 3

Memory: 2

ALU: 1

Mult: 1

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width.

The ADSP-2191M does not implement SIMD.

## Benchmarked Routines []

FFT

Discrete Cosine Transform

GSM Codec

IIR

Linear Predictive Coding

Biquad IIR filter

FIR

Transversal Filter

DTMF encoder/decoder

Sine approximation

Companding

Division

Arctangent

Log

Square root

Euclidean distance

## References

[1] ADSP-21362/ADSP-21363/ADSP-21364/ADSP-21365/ADSP-21366 SHARC Data Sheet, Rev C, September 2007.

[2] ADSP-2136x SHARC® Processor Programming Reference, Rev 1.0, March 2007.

[3] http://www.bdti.com/procsum/adi219x.htm

[4] http://www.analog.com/processors/adsp/technicalLibrary/codeExamples/applicationsHandbook.html

# Pentium IV 661

## Information from General Conventions

DSP ID: P4-661

## Information from DataSheet [1,2,3]

Native Precision: 128 (enabled with SSE2 which extends all MMX 64-bit ops to 128-bits)

Clock Rate (MHz):  3600

The P4-661 is a member of the Pentium IV family. This means it supports the following technologies:

- Intel NetBurst microarchitecture

- streaming SIMD extensions

  - SSE

  - SSE2

  - SSE3

- Hyperthreading

- Intel® EM64T (http://developer.intel.com/technology/architecture-silicon/intel64/index.htm)

Note this means it does not support Virtualization Technology (which came in with 662), SSE4, and multi-core technologies. In terms of instruction sets (see Table 5-1 in [2]), this means the P4-661 supports the following instruction sets:

- General purpose

- x87 FPU

- x87 PU and SIMD State Management

- MMX (integer SIMD)

- SSE Extensions (single float SIMD)

- SSE2 Extension (double float SIMD)

- SSE3 Extensions (13 new instructions)

- IA-32e mode

- System Instructions

The NetBurst Microarchitecture supports branch prediction, superscalar operation (three instructions can be retired per cycle), out-of-order execution cores, supports fixed and floating-point processing, and binary coded decimal. It consists of 2 fast ALUs (clock at twice main clock rate), 1 slow ALU (does multiplication, shifting), one data load unit, one data store unit, a floating point ALU (float and fixed SIMD) and a floating point memory unit (float/fixed Move). Some support is given for looping and conditional branching (jumping). An extensive library of floating point operations are provided but are ignored because they consume numerous cycles (latencies for most Intel instructions are available at http://modseven.de/dictionary.php).

SIMD operations are somewhat different than other processors’ SIMD operations in that there can be some staggered latency in computing all of the results (See http://swox.com/doc/x86-timing.pdf). However, in general, the P4-661’s SIMD pipelined throughput is approximately similar to what we get from other processors. Note that SIMD multiplication only goes down to 16 bits, so we will model the processor as having a minimum SIMD width of 16-bits, even though it can do a SIMD add down to 8-bit words (e.g., PADDB). As the SSE2 instructions extend these SIMD operations to work on the 128 bit XMM registers, significant parallelism is possible from SIMD.

Hyper-threading is used to manage multiple threads by maintaining independent architectural states (thereby effecting multiple logical cores) within the same processor core. Each logical processor consists of a full set of IA-32 data registers (including interrupt and debug registers) so multiple local copies of local processor memory are maintained, but the actual execution resources are not increased and are instead shared between the logical processors. Whether or not hyperthreading improves efficiency is irrelevant to this project (there’s arguments on both sides and it likely depends on the specific application) as for our purposes hyperthreading will not impact execution predictions as the modules we map are assumed to keep execution resources fully occupied. It should also be noted that the shared caching used in hyperthreading can be used to by a malicious thread to steal cryptographic keys from another thread.

## Peak Power Calculation

[2] gives a peak power measurement (thermal design power) of 86 W or 86,000 mW.

## Characteristics

Fixed

Float

Branch prediction

Rounding

Saturation

Binary coded decimal

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2 called “base-plus index addressing” by Intel)

## Modifiers

AVG

The P4-661 supports an averaging operation in its SIMD unit.

BPOS

The P4-661 supports numerous different conditional branches.

COND_MOV

All memory operations can be executed conditionally

EXTRACT

Due to its implementation of various bit/byte manipulation instructions (e.g., BSF, BTS), the P4-661 will be considered as implementing the EXTRACT capability.

LOOP

The P4-661 supports a unique loop structure in which the loop instruction compares the loop counter with zero, branches as needed and decrements the counter.

MAX

In its SIMD unit, the P4-661 supports instructions whereby words are compared and the greater (lesser) is stored.

SAD

Sum of absolute differences. Useful in a vector distance metric.

VECT

The P5-661 has an instruction (PMADD) wherein pairs of words are multiplied and added. For four pairs of words, 2 results are made. This is slightly different from a complex multiplication and is more akin to an operation to support a dot product of two vectors.

## VLIW Information

The P4-661 can issue four instructions in a cycle and each of its fast ALUs can complete two operations per clock cycle. However, the only factor that matters is that only 3 instructions can complete per cycle. Note that only one unit is available for SIMD arithmetic operations (and one for SIMD memory operations). This will have a bigger impact on multiplications than ALU operations (control sorts of operations could easily be going on in parallel on one of the other three ALUs) so we’ll model this as follows.

VLIW_flag: 1

VLIW_max: 3

VLIW Memory: 2

VLIW ALU: 2

VLIW Mult: 1

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The P4-661 implements SIMD via its MMX and SSE extensions which allow operations down to the byte level on 128 XMM registers for additions and down to the integer word level (16-bit) for multiplications. Because of the importance of multiplications to our process, we’ll model the P4-661 as supporting SIMD down to a minimum word size of 16 with a maximum width of 128. Note that every SIMD instruction takes multiple cycles to complete, but these are pipelined so that in general a throughput of one SIMD instruction per cycle is achieved (see http://www.tommesani.com/P4MMX.html)

SIMD_flag: 1

SIMD_min: 16

## Benchmarked Routines

There exists standard library code, but it’s not focused on DSP-type operations (e.g., memcpy, strlen).

## References

[1] Intel® Pentium® 4 Processor 6x1Δ Sequence Datasheet, Document Number: 310308-002, December 2006.

[2] http://processorfinder.intel.com/Details.aspx?sSpec=SL94V

[3] Intel® 64 and IA-32 Architectures Software Developer's Manual Volume 1: Basic Architecture

[4] Intel® 64 and IA-32 Architectures Software Developer's Manual Volume 2A: Instruction Set Reference, A-M, February 2008.

[5] Intel® 64 and IA-32 Architectures Software Developer's Manual Volume 2B: Instruction Set Reference, N-Z, February 2008.

G. Hinton, D. Sager, M. Upton, D. Boggs, D. Carmean, A. kyker, P. Roussel, The Microarchitecture of the Pentium 4 Processor, Intel Technology Journal Q1, 2001, pp. 1-13

# MPC8540

## Information from General Conventions

DSP ID: MPC8540

## Information from DataSheet [1-5]

Native Precision: 64 (really 32, but needed to model the way registers and data accesses are implemented)

Clock Rate (MHz):  1000 (this is the highest value)

The MPC8540 combines a 32-bit PowerPC processor core (e500) with extensive peripherals for supporting networking protocols (e.g., RapidIO, ethernet). For our purposes, we’re concerned with the processing core – the e500. The e500 implements an enhancement on the PowerPC Book E instruction set architecture and can complete 2 instructions per clock cycle as implemented over it five functional units – a branch unit, a load/store unit, a multiple cycle unit, and two single cycle units. For our purposes, the two single-cycle units will be modeled as ALUs and the multiple cycle unit as a MAC.

The three computational units are extended via a number of auxiliary processing units (APU) which Integer select APU, performance monitor APU, signal processing engine APU (SPE APU) embedded floating point APUs (scalar and vector), cache block lock and unlock APU, and a machine check APU. It is perhaps more convenient to view these APUs as extensions to the instruction sets supported by the three computational units as opposed to stand-alone units as their operation is integrated into the computational units’ pipelines.

The e500 is a RISC architecture which has taken significant steps to provide capabilities to synchronize operations and limit memory race conditions. Additionally, most instructions can be completed in a single cycle and those that do not are pipelined so that other than some initial latency (which we’re ignoring other processors), so it is likely that most of the timing irregularities of RISC processors will not be experienced on the MPC8540 (cannot be certain without actual handcoding, however).

Additionally, the SPE APU defines instructions which permit 32-bit SIMD operation on 64-bit registers for effectively all multiplication, memory, and ALU operations. It should be noted that the MPC8540 documentation strongly discourages the use of SPE instructions because they will be deprecated in future releases. The documentation suggests using the libmoto_e500 library instead. However, from emails with Freescale, it was learned that the libmoto_e500 library no longer exists and has been replaced by the libcsfl_e500 library, but it only supports vectorized memory operations (e.g., memcpy). The integer select unit supports very short branching where the condition register has been previously set. Numerous different branching options are allowed (including BDEC, BPOS).

## Peak Power Calculation

Table 4 in [1] gives a peak power consumption of 15,900 mW according to the configuration assumed in this mapping (1000 MHz).

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Float

RISC

Branch prediction

Rounding

Saturation

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

Reverse

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The MPC8540 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

BDEC

The MPC8540 supports an operation whereby control branches if the counter register is positive and decrements. To make behavior this conform to other modeled processors, these are broken into BPOS and BDEC steps.

BITR

Reverses the bits of a register in a single instruction. Technically, however, the MPC8540 is implementing this a bit-reversed carry instruction.

EXTRACT

Detailed bit manipulation is possible via various rotate and mask insert instructions.

ISEL

An instruction wherein one of two different registers is moved into a register based on a condition bit (set previously – so unlike a MAX operation).

MAC

A single-cycle multiply and accumulate.

## VLIW Information

The MPC8540 can complete two instructions in a cycle and has two ALUs, a multiplier, and a data memory unit. This will be modeled as follows.

VLIW_flag: 1

VLIW_max: 2

VLIW Memory: 1

VLIW ALU: 2

VLIW Mult: 1

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The MPC8540 implements SIMD via its SPE APU wherein 64 bit registers are treated as pairs of 32-bit registers and the same operation is performed. This will be modeled as follows.

SIMD_flag: 1

SIMD_min: 32

## Benchmarked Routines

There exists standard library code, but it’s not focused on DSP-type operations and instead does GPP-type operations (e.g., memcpy, strlen).

## References

[1] MPC8540 Integrated Processor Hardware Specifications, MPC8540EC

Rev. 4.1, July 2007.

[2] MPC8540 PowerQUICC III™ Integrated Host Processor Reference Manual, MPC8540RM Rev. 1 July 2004.

[3] http://www.arm.com/products/CPUs/ARM968E-S.html PowerPC™ e500 Core Family Reference Manual, E500CORERM Rev. 1, April, 2005.

[4] Book E: Enhanced PowerPCÔ Architecture Version 1.0 May 7, 2002

[5] EREF: A Reference for Freescale Book E and the e500 Core, Revision 2.0, January 2004.

# DSP56321

The following provides details on how parameters for the data file for the DSP56321 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: DSP56321

## Information from DataSheet [],[2]

Native Precision: 24

Clock Rate (MHz):  275

The DSP56321 combines the DSP5600 core with an Enhanced Filter Coprocessor. The DSP5600 core includes a Data ALU unit which has a MAC, bit field unit, and an ALU, and a data address generation unit which supports two word loads in parallel to an operation by the Data ALU unit. Primarily a 24-bit processor, it also supports 16-bit operations and supports conditional execution of all operations (but not when performing memory operations in parallel). It also supports zero-overhead looping (which Freescale calls “hardware do-loops”).

Note that the enhanced filter coprocessor [3] runs at the same clock rate as the DSP56321 and supports various filter structures. However, it does not support instruction based processing and thus will be treated as a coprocessor which means that its cycle benefits would only appear in benchmarked routines.

## Peak Power Calculation

[4] states that the DSP56371 (same core) typically consumes about 124 mW in terms of core and memory consumption. However the DSP56371 operates at 180 MHz and 1.25 V and the DSP56321 operates at 275 MHz at 1.6 V. The DSP56371 measurement can be applied to the DSP56321 by scaling (275/180)*(1.6/1.25)^2 or about 2.5. Thus the typical internal core / memory power can be estimated as 2.5*124 mW = 310 mW.

This, however, misses out on the power consumption of the Enhanced Filter Coprocessor. Loosely, well model this as consuming the same fraction of internal power as the ARM1136J(F)-S consumed by the floating point coprocessor which we had estimated at 17% of internal power, or 310*0.17 = 53 mW for a total of 363 mW.

Assuming this represents 50% loading and the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 363 mW

dynamic = 1.54 static

static = 205 mW

peak dynamic = 316 mW

This gives a total power consumption of 620 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [1].

Fixed

Round

Saturation arithmetic

Circular

Reverse (called reverse-carry in FreeScale lingo)

Internal Scaling (automated shifting)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The DSP56321supports conditional execution of all instructions, including on positive values.

COND_EXEC

Instructions are executed based on evaluation of a condition – simplifies short control structures. The DSP56321 supports conditional execution of its operations; however this cannot be done with parallel memory accesses. This, however, will be ignored.

INDEX

Indexed addressing, e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2.

MAC

A single-cycle multiply and accumulate.

VSL (Viterbi Shift Left)

A process by which a register is shifted left and an input 1 or 0 is appended to the right most bit. Useful for keeping track of paths (saves an instruction).

ZOL

According to [], the DSP56321 implements hardware do-loops which we’ll model as implementing zero-overhead looping.

## VLIW Information

The DSP56321 can do two memory accesses while performing an arithmetic operation. This will be modeled as follows.

VLIW_flag: 1

VLIW_max: 3

VLIW Memory: 2

VLIW ALU: 1

VLIW Mult: 1

This is slightly misleading as it implies that a MULT, ALU, and Memory could simultaneously execute, but this is not the case.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The DSP56321 does not support SIMD.

## Benchmarked Routines

Appendix B in [2] provides numerous programs for benchmarking the operation of a DSP in the DSP56300 family, but does include execution statistics to go with this.

## References

[] DSP56321 Technical Data, Rev. 11, 2/2005

[2] DSP56300 Family Manual: 24-Bit Digital Signal Processors, DSP56300FM Rev. 5, April 2005

[3] T. Redheendran, Programming the DSP56300 Enhanced Filter Coprocessor (EFCOP), APR39, Rev 1, August 2005.

[4] http://www.arm.com/products/CPUs/ARM968E-S.htmlhttp://www.bdti.com/procsum/mot563xx.htm

# DSP56F8367

The following provides details on how parameters for the data file for the DSP56321 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: DSP56F8367

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz):  60

The DSP56F8367 is built around the DSP56800E core which includes a Data ALU unit which has a MAC, bit field unit, and an ALU, and a data address generation unit which supports two word loads in parallel to an operation by the Data ALU unit. Technically, this parallelism is primarily single memory load (independent of processing), but has significantly many instructions for which dual memory load is possible that we’ll consider it to be dual. It also supports zero-overhead looping (which Freescale calls “hardware do-loops”).

Unlike the DSP56321, it does not implement the VSL instruction [2], nor have a filter coprocessor, nor do bit-reverse, and does not support general conditional execution of operations (though moves and branches can be conditional).

## Peak Power Calculation

[3] states that the DSP56F8011 (which uses the same core as the DSP56F8367) consumes a maximum of 59 mA. According to [4], the DSP56F8011 operates at a clock rate of 32 MHz and a voltage of 3.6 V for a total power of 212 mW. [5] notes that the DSP56F8367 operates at 2.5 V and a clock rate of 60 MHz. Scaling by (60/30)*(2.5/3.6)2 gives 205 mW.

Assuming this represents 50% loading and the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 205 mW

dynamic = 1.54 static

static = 116 mW

peak dynamic = 179 mW

This gives a total power consumption of 394 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [2].

Fixed

Round

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

Circular (calls modulo)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The DSP56321 supports conditional branching on positive values.

MAC

A single-cycle multiply and accumulate.

ZOL

According to [2], the DSP56321 implements hardware do-loops which we’ll model as implementing zero-overhead looping.

## VLIW Information

The DSP56F8367 can do two memory accesses while performing an arithmetic operation. This will be modeled as follows.

VLIW_flag: 1

VLIW_max: 3

VLIW Memory: 2

VLIW ALU: 1

VLIW Mult: 1

This is slightly misleading as it implies that a MULT, ALU, and Memory could simultaneously execute, but this is not the case.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The DSP56F8367 does not support SIMD.

## Benchmarked Routines

Appendix B in [2] provides numerous programs for benchmarking the operation of a DSP in the DSP56300 family, but does include execution statistics to go with this.

## References

[1] 56F8367/56F8167 Data Sheet, Rev 8, January 2007.

[2] DSP56800E Reference Manual, DSP56800ERM, Rev. 2.16, Nov 2005.

[3] C. Wu, “Understanding of 56F800E DSC Motor Control Peripherals and Applications,” PZ308, November 2007 Available online: http://mcuol.com/download/upfile/PZ308.pdf

[4]http://www.freescale.com/webapp/sps/site/prod_summary.jsp?code=56F8011&nodeId=01279562921379

[5]http://www.freescale.com/webapp/sps/site/prod_summary.jsp?code=56F8367&nodeId=01279562921379&tab=Buy_Parametric_Tab&fromSearch=false#2

# MSC7116

The following provides details on how parameters for the data file for the MSC8101 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: MSC7116

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz):  266

The MSC7116 is based on a single SC1400 core for which little direct documentation exists. However as noted in [2]:

“The SC140 and SC1400 cores are functionally identical, and the information in this document applies to both cores. For simplicity, the SC140 core is referenced throughout this application note”

Thus characteristics are taken from the SC140 reference manual [3] as opposed to a non-existent SC1400 reference manual.

The SC140 has 4 Data ALUs, each of which contains an ALU unit and a MAC unit. Additionally, the SC140 contains a Bit-field Unit and 2 data address generation units which combined can perform 2 64-bit moves per cycle. Each of these six units can operate independently to execute a total of six instructions simultaneously. The SC140 also supports hardware loops (called DO loops) and can manage a co-processor. The MSC7116 lacks such a co-processor, but like many chips, it has dedicated circuitry for network interfaces (in this case Ethernet).

## Peak Power Calculation

The data sheet [1] provides an example power calculation which gives the following values:

Peripheral power = 15.3 mW

Static (leakage) power = 64 mW

External 326.3 mW (memory)

Core = 287 mW

Taking this core value to be “typical” (i.e., 50%), we double for a peak of 574 mW for a total of 1052 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [2].

Fixed

Round

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

Circular (calls modulo)

Reverse (called reverse carry)

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ADDSUB

The SC140 includes instructions (e.g., ADD20 which we’ll model as an ADDSUB capability as it otherwise does not support SIMD.

BPOS

Using its ALU units, the SC140 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

EXTRACT

Using its multiplier units, the SC140 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

IFF

An instruction, which if true, results in execution of remaining instructions in a group. This will not be modeled as in the current model VLIW + a branching instruction effects that behavior.

MAC

A single-cycle multiply and accumulate.

MAX

Using its ALU units, the SC140 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

MAX2

Like the MAX operation, but performed on two word pairs simultaneously.

MEM4

The 64-bit loads effectively act as a quad load.

VSL (Viterbi Shift Left)

A process by which a register is shifted left and an input 1 or 0 is appended to the right most bit. Useful for keeping track of paths (saves an instruction).

ZOL

According to [], the SC140 implements hardware do-loops which we’ll model as implementing zero-overhead looping.

## VLIW Information

Each of the 4 data ALUs and the 2 data address generator units can operate independently.

VLIW_flag: 1

VLIW_max: 6

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 4

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The SC140 does not support SIMD.

## Benchmarked Routines

None available??

## References

[1] MSC7116 Document Number: MSC7116 Rev. 11, July 2007.

[2] http://www.freescale.com/files/dsp/doc/app_note/AN2715.pdf?fpsp=1&WT_TYPE=Application%20Notes&WT_VENDOR=FREESCALE&WT_FILE_FORMAT=pdf&WT_ASSET=Documentation

[3] SC140 DSP Core Reference Manual Revision 4.1, September 2005

[4] MSC711X Reference Manual: MSC711xRM Rev. 1, November 2006.

[5] D. Simon, MSC711x Overview, AN3056, Rev. 0, December 2005.

# MSC8101

The following provides details on how parameters for the data file for the MSC8101 were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: MSC8101

## Information from DataSheet []

Native Precision: 16

Clock Rate (MHz):  300

The MSC8101 is based on a single SC140 core. The SC140 has 4 Data ALUs, each of which contains an ALU unit and a MAC unit. Additionally, the SC140 contains a Bit-field Unit and 2 data address generation units which combined can perform 2 64-bit moves per cycle. Each of these six units can operate independently to execute a total of six instructions simultaneously. The SC140 also supports hardware loops (called DO loops) and can manage a co-processor.

The MSC8101 includes an enhanced filter coprocessor [3] which runs at the same clock rate as the MSC8101 and supports various filter structures. However, it does not support instruction based processing and thus will be treated as a coprocessor which means that its cycle benefits would only appear in benchmarked routines. Additionally, the MSC8101 contains dedicated circuitry for most network interfaces (e.g., IP, ATM Ethernet).

## Peak Power Calculation

The data sheet [1] provides an example power calculation assuming a core clock rate of 300 MHz and gives the following values:

Core = 450 mW

PCM = 163 mW

PSIU = 41 mW

PI/O = 67 mW

Taking this core value to be “typical” (i.e., 50%), we double for a peak of 900 mW for a total of 1171 mW.

## Characteristics:

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

ADDSUB

The SC140 includes instructions (e.g., ADD2 which we’ll model as an ADDSUB capability as it otherwise does not support SIMD.

BPOS

Using its ALU units, the SC140 supports an operation whereby control branches if a register is positive (equivalent to a branch and a comparison).

EXTRACT

Using its multiplier units, the SC140 supports several operations which collectively simplify the process of extracting/packing bit fields into registers. In general, this is treated as cutting bit manipulation cycles in half, but the exact savings by supporting detailed bitfield manipulation operations will vary from component to component.

IFF

An instruction, which if true, results in execution of remaining instructions in a group. This will not be modeled as in the current model VLIW + a branching instruction effects that behavior.

MAC

A single-cycle multiply and accumulate.

MAX

Using its ALU units, the SC140 supports an operations whereby half-words (or quarter-words) are compared and the greater (or lesser) half (quarter) word stored, thereby combining compare and move operations into a single cycle.

MAX2

Like the MAX operation, but performed on two word pairs simultaneously.

MEM4

The 64-bit loads effectively act as a quad load.

VSL (Viterbi Shift Left)

A process by which a register is shifted left and an input 1 or 0 is appended to the right most bit. Useful for keeping track of paths (saves an instruction).

ZOL

According to [], the SC140 implements hardware do-loops which we’ll model as implementing zero-overhead looping.

## VLIW Information

Each of the 4 data ALUs and the 2 data address generator units can operate independently.

VLIW_flag: 1

VLIW_max: 6

VLIW Memory: 2

VLIW ALU: 4

VLIW Mult: 4

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. The SC140 does not support SIMD.

## Benchmarked Routines

None available??

## References

[1] MSC8101 Data Sheet Rev. 18, August 2005.

[2] SC140 DSP Core Reference Manual Revision 4.1, September 2005 .

[3] MSC8101 Reference Manual: MSC8101RM Rev. 4, December 2005.

# ARM 968E-S Data Sheet

The following provides details on how parameters for the data file for the ARM968E-S were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ARM968E-S

## Information from DataSheet []

Native Precision: 32

Clock Rate (MHz):  530

The ARM 968E-S is a member of the ARM9 Thumb family and implements the ARMv5TE architecture. It supports the 32-bit ARM instruction set and the 16-bit Thumb instruction set. While the amount of time required to execute cycles is dependent on the following instructions (interlock), we’ll in general ignore this except for the fact that most cycle savers do not actually save cycles (e.g., the double word store instruction takes 2 cycles instead of 1).

In general, it doesn’t support detailed bit manipulation, but it does allow you to count leading zeros.

## Peak Power Calculation

For operation at 530 MHz, [3] gives a core power consumption of 0.11 mw/MHz as typical power consumption or 58.3 mW. Assuming this represents 50% loading and that the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 58.3 mW

dynamic = 1.54 static

static = 32.9 mW

peak dynamic = 50.7 mW

This gives a total power consumption of 183 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [].

Fixed

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

RISC

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The ARM968E-S supports conditional execution of its branches.

COND_EXEC

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ARM968E-S can be conditionally executed, so it is modeled as supporting conditional execution.

MAC

A “single-cycle” multiply and accumulate. Note the instructions on ARMs are generally not single-cycle, but we’re not modeling to that level of detail.

## VLIW Information

In ARM-speak, VLIW is called Instruction Level Parallelism (ILP). The ARM 968E-S does not support ILP.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. ARM began to support SIMD with ARM11.

## Benchmarked Routines

None available??

## References

[1] ARM9E-S Core™ Revision: r2p1Technical Reference Manual, July 2004, DDI 0240B.

[2] ARM Architecture Reference Manual, July 2000, DDI0100E.

[3] http://www.arm.com/products/CPUs/ARM968E-S.html

[4] J. Goodacre, A. Sloss, “Parallelism and the ARM instruction set architecture,” Computer, Volume 38,  Issue 7,  July 2005 Page(s):42 – 50.

# ARM 1020E Data Sheet

The following provides details on how parameters for the data file for the ARM1020E were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ARM1020E

## Information from DataSheet []

Native Precision: 32

Clock Rate (MHz):  325

The ARM1020E is a member of the ARM10 Thumb family and implements the ARMv5TE architecture. It supports the 32-bit ARM instruction set and the 16-bit Thumb instruction set. While the amount of time required to execute cycles is dependent on the following instructions (interlock), we’ll in general ignore this except for the fact that most cycle savers do not actually save cycles and that [1] says “it delivers a peak throughput approaching one instruction per cycle.”

For our modeling purposes the primary difference between the ARM1020E and the ARM968E-S is the independent load/store unit which can load/store 2 registers per cycle in parallel to data processing.

## Peak Power Calculation

For operation at 325 MHz, [2] gives a core power consumption of 0.6 mW/MHz as typical power consumption or 195 mW. Assuming this represents 50% loading and the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 195 mW

dynamic = 1.54 static

static = 110 mW

peak dynamic = 170 mW

This gives a total power consumption of 379 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [1].

Fixed

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

RISC

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The ARM1020E supports conditional execution of its branches.

Conditional Execution

Instructions are executed based on evaluation of a condition – simplifies short control structures. All instructions on the ARM1020E can be conditionally executed, so it is modeled as supporting conditional execution.

MAC

A “single-cycle” multiply and accumulate. Note the instructions on ARMs are generally not consistently single-cycle (depends on following instruction), but we’re not modeling to that level of detail.

## VLIW Information

In ARM-speak, VLIW is called Instruction Level Parallelism (ILP). The ARM1020E does not support ILP, but the load/store unit can operate independently of the Integer processing unit. This is modeled as follows.

VLIW_flag: 1

VLIW_max: 3

VLIW Memory: 2

VLIW ALU: 1

VLIW Mult: 1

This is slightly misleading as it implies that a MULT, ALU, and Memory could simultaneously execute, but this is not the case.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. ARM began to support SIMD with ARM11.

## Benchmarked Routines

None available??

## References

[1] ARM9E-S Core™ Revision: r1p7Technical Reference Manual, June 30 2003, DDI 0177E.

[2] http://www.arm.com/products/CPUs/ARM1020E.html

[3] J. Goodacre, A. Sloss, “Parallelism and the ARM instruction set architecture,” Computer, Volume 38,  Issue 7,  July 2005 pp. 42-50.

# ARM1136J(F)-S Data Sheet

The following provides details on how parameters for the data file for the ARM1136J(F)-S were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ARM1136J(F)-S

## Information from DataSheet []

Native Precision: 32

Clock Rate (MHz):  620

The ARM1136J(F)-S supports the ARM6 instruction set, the Thumb instruction set, and the Jazelle (Java) instruction set (the J in ARM1136J(F)-S). Additionally, it supports DSP extensions in which SIMD is supported (to 16). The basic ARM consists of an ALU, a MAC, a shifter and a load/store unit which can load/store two words per cycle independently of the ALU/MAC operation.

Additionally, it also has a dedicated vector float coprocessor (the F in ARM1136J(F)-S). The VFP11 has 3 independent processes – a MAC pipeline, a load/store pipeline, and a divide and square root pipeline. These actually are single-cycle processes (except for divide and square), LS can supply two single-precision operands. The divide and square root does 2 instructions, divide and square root. This will be treated as a coprocessor potentially modeled in benchmarked functions as none of our applications require these instructions.

The combination of ARM and VFP11 will be treated as a VLIW architecture capable of supporting 4 simultaneous memory operations, 2 MAC and/or two ALU.

## Peak Power Calculation

For operation at 620 MHz, [2] gives a core power consumption of 0.45 mW/MHz as typical power consumption or 279 mW. Assuming this represents 50% loading and the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 279 mW

dynamic = 1.54 static

static = 158 mW

peak dynamic = 243 mW

This gives a total power consumption of 500 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [1].

Fixed

Float

Round

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

Note we’re not calling this a RISC processor because we expect the majority of the processing to occur on the floating-point co-processor.

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The ARM1136J(F)-S supports conditional execution of all instructions, including on positive values.

Conditional Execution

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ARM1136J(F)-S can be conditionally executed, so it is modeled as supporting conditional execution.

MAC

A “single-cycle” multiply and accumulate. Note the instructions on ARMs are generally not consistently single-cycle (depends on following instruction), but we’re not modeling to that level of detail.

SAD

Sum of absolute differences. Unable to find decent documentation on this, so will be ignored.

## VLIW Information

In ARM-speak, VLIW is called Instruction Level Parallelism (ILP). The combination of ARM and VFP11 will be treated as a VLIW architecture capable of supporting 4 simultaneous memory operations, 2 MAC and/or two ALU.

VLIW_flag: 1

VLIW_max: 6

VLIW Memory: 4

VLIW ALU: 2

VLIW Mult: 2

This is slightly misleading as it implies that a 2 MULT, 2 ALU, and 2 Memory could simultaneously execute, but this is not the case.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. ARM began to support SIMD with ARM11[3]. The ARM1136J(F)-S implements SIMD down to 8-bit in its ALUs (in add/sub routines) and down to 16 bits in its multiplier. We are ignoring the 8-bit capability in this model.

SIMD_flag: 1

SIMD_min: 16

## Benchmarked Routines

None available??

## References

[] ARM1136JF-S™ and ARM1136J-S™ Revision: r1p5 Technical Reference Manual, DDI0211J, July 6, 2007.

[2] VFP11™ Vector Floating-point Coprocessor for ARM1136JF-S processor r1p5 Technical Reference Manual, DDI 0274H July 6, 2007.

[3] http://www.arm.com/products/CPUs/ARM1020E.html

[4] J. Goodacre, A. Sloss, “Parallelism and the ARM instruction set architecture,” Computer, Volume 38,  Issue 7,  July 2005 pp. 42-50.

# ARM1136J-S

The following provides details on how parameters for the data file for the ARM1136J(F)-S were generated for the Tool for Automating Estimation of DSP Resource Statistics for Waveform Components.

## Information from General Conventions

DSP ID: ARM1136J-S

## Information from DataSheet []

Native Precision: 32

Clock Rate (MHz):  620

The ARM1136J-S supports the ARM6 instruction set, the Thumb instruction set, and the Jazelle (Java) instruction set (the J in ARM1136J-S). Additionally, it supports DSP extensions in which SIMD is supported (to 16). The basic ARM consists of an ALU, a MAC, a shifter and a load/store unit which can load/store two words per cycle independently of the ALU/MAC operation.

Unlike the ARM1136J(F)-S, it does not have a dedicated vector float coprocessor.

## Peak Power Calculation

ARM [2] does not give a typical estimate for the ARM1136J-S, but it does for without memory cache which decreases core power consumption of 0.45 mW/MHz to 0.37 mW/MHz which we’ll assume for operation without the floating point unit. This gives a typical power consumption or 229 mW at 620 MHz. Assuming this represents 50% loading and the 99mW of the ADSP-21632 and its ratio of dynamic to static power consumption remains at 1.54 at peak loading, then we have the following system of equations.

0.5 dynamic + static = 229 mW

dynamic = 1.54 static

static = 130 mW

peak dynamic = 180 mW

This gives a total power consumption of 409 mW.

## Characteristics

Characteristics are DSP capabilities which might be assumed by a programmer when implementing a waveform component. The following were identified via a review of [1].

Fixed

Round

Saturation arithmetic

Indexed addressing (e.g., *R1++R2 where value in R1 is loaded and then R1 is offset by R2)

RISC

## Modifiers

Modifiers are instructions which permit the simultaneous execution of two or more operations in a single instruction cycle (excepting saturation and sign manipulation).

BPOS

The ARM1136J-S supports conditional execution of all instructions, including on positive values.

COND_EXEC

Instructions are executed based on evaluation of a condition – simplifies short control structures. Many instructions on the ARM1136J-S can be conditionally executed, so it is modeled as supporting conditional execution.

MAC

A “single-cycle” multiply and accumulate. Note the instructions on ARMs are generally not consistently single-cycle (depends on following instruction), but we’re not modeling to that level of detail.

SAD

Sum of absolute differences. Unable to find decent documentation on this, so will be ignored.

## VLIW Information

In ARM-speak, VLIW is called Instruction Level Parallelism (ILP). On the ARM1136J-S the load/store unit can operate independently of the Integer processing unit. This is modeled as follows.

VLIW_flag: 1

VLIW_max: 3

VLIW Memory: 2

VLIW ALU: 1

VLIW Mult: 1

This is slightly misleading as it implies that a MULT, ALU, and Memory could simultaneously execute, but this is not the case.

## SIMD Information

Single-Instruction-Multiple-Data (SIMD) is a characteristic of some DSP architectures wherein a single instruction can be simultaneously applied to two or more words smaller than the native data width. ARM began to support SIMD with ARM11[3]. The ARM1136J-S implements SIMD down to 8-bit in its ALUs (in add/sub routines) and down to 16 bits in its multiplier. We are ignoring this capability in this model.

SIMD_flag: 1

SIMD_min: 16

## Benchmarked Routines

None available??

## References

[1] ARM1136JF-S™ and ARM1136J-S™ Revision: r1p5 Technical Reference Manual, DDI0211J, July 6, 2007.

[2] VFP11™ Vector Floating-point Coprocessor for ARM1136JF-S processor r1p5 Technical Reference Manual, DDI 0274H July 6, 2007.

[3] http://www.arm.com/products/CPUs/ARM1020E.html

[4] J. Goodacre, A. Sloss, “Parallelism and the ARM instruction set architecture,” Computer, Volume 38, Issue 7, July 2005 pp. 42-50.

This classification step is the greatest source of inaccuracy in the estimation process. More accurate estimates could be formed by creating a separate execution model for each DSP, but this would be dramatically more complicated and time-consuming and would require an entirely different software architecture for the computational engine.

In practice, this is accomplished by stopping the carry operation at 8-bit boundaries instead of propagating the carry bit through the entire 32 bits.

Details to be inserted as components are identified

Details to be inserted as components are identified

Note that there are some other instructions which effectively pipeline sub-instructions and reduce the number of required fetches from program memory, but these types of modifiers are not the intended study of this project. For example, the TMS320C6455-850 implements a complex multiplication instruction which performs the our multiplications involved with a complex multiplication. However, this is performed over four cycles so no cycles are saved by using this instruction.

Details to be inserted as components are identified

Note that there are some other instructions which effectively pipeline sub-instructions and reduce the number of required fetches from program memory, but these types of modifiers are not the intended study of this project. For example, the TMS320C6455-850 implements a complex multiplication instruction which performs the our multiplications involved with a complex multiplication. However, this is performed over four cycles so no cycles are saved by using this instruction.

Details to be inserted as components are identified

Note that there are some other instructions which effectively pipeline sub-instructions and reduce the number of required fetches from program memory, but these types of modifiers are not the intended study of this project. For example, the TMS320VC5502-300 implements an LMS instruction which occurs over numerous cycles and no cycles are saved by using this instruction versus hand-coding the equivalent operations.

Details to be inserted as components are identified

Details to be inserted as components are identified

Details to be inserted as components are identified

Details to be inserted as components are identified

The CLU implements several unique instructions including TMAX, dispreading, cross correlation, add-compare-select, and trellis operations. These, however, should be captured in relevant benchmarked routines.

Details to be inserted as components are identified

Details to be inserted as components are identified