Example Applications (Task 2.4)

for the

Tool for Automating Estimation of DSP Resource Statistics for Waveform Components

Submitted under Subcontract FP-19738-430292

An Integrated Tool for SCA Waveform Development, Testing, and Debugging and a Tool for Automated Estimation of DSP Resource Statistics for Waveform Components

Version 1.0

| Version | Summary of Changes | Date |
| --- | --- | --- |
| 1.0 (JN) | Initial Release | 6/11/08 |

# Document Overview

This document provides two example uses of the PCET tool and gives validation results on the performance of the tool.

# Implementation Comparisons

PCET allows for quick comparisons as to how varying a single components parameters will impact the use of DSP resources. For instance, we varied the length of the real filter component over 16, 32, 64, and 128 taps assuming 1 us is available to implement the filter. This led to the following results. Note that because of the looped structure, changing these parameters had little effect on program memory size, but did impact data memory (ultimately same total number of bytes per processor). Cycles varied significantly and power varied significantly with the traditional low power processors (C54/55) requiring the least power. The traditional power hog, the Pentium IV, was not the largest energy consumer (though it was quite high) because it's massive SIMD capabilities permitted it to execute its operations in very short periods of time. Note that the modeled filter implementation assumes circular buffering which is not supported on the modeled ARM processors.

Figure 1: Cycle Estimates

Figure 2: Time requirements

Figure 3: Data Memory Estimates

Figure 4: Program Memory Estimates

Figure 5: Power Estimates

# Feasibility Studies

PCET can also be used to generate rapid estimations as to which platforms could support a desired waveform. Using the existing components we mocked up a waveform that is similar to an OFDM receiver with an FFT, interpolator (standing in for a decimator), filter, and QAM demodulator. This led to the following results.

Technically, none of the DSPs were able to support the waveform (OFDM is traditionally very hard to do on a processor), but the Pentium was only because it lacked the reverse bit addressing mode assumed in the FFT component. Coding a new FFT module which does not assume bit-reverse addressing would add a good number of cycles to perform the requisite shuffling, but it should still fit on the Pentium. Also note that while the Pentium executed the Viterbi the fastest, the C64s and the Sharc processors required the least amount of program memory. This implies that the massive potential parallelism of the Pentium led to faster execution than the C64 and Sharc's specialized instruction set. However, this also led to the Pentium consuming an order of magnitude more power. Finally, note that the Viterbi decoder could be implemented with different assumptions such that the Sharc and C64 Viterbi coprocessors could be used. However, this too would require coding up an addition component for PCET.

Figure 6: Cycle Estimates

Figure 7: Time estimates

Figure 8: DataMemory Estimates

Figure 9: Program Memory Estimates

Figure 10: Estimated Power Consumption

# PCET Validation

Some testing of PCET was also done to evaluate how well abstract high level models of components and DSPs would match up with reality. Comparisons were made to similar library code and implementation measurements were also made.

## Library Validation

To make comparisons to library implementations, the existing documentation of DSP libraries were reviewed and examined for similarity to the assumptions used in modeling components. However, there are still differences in implementation approach between what is assumed in the PCET module and the library reported values. The following also ignore overhead (e.g., stack management) which is not modeled in the components.

Real Filter

Real Filters were varied for filter lengths 15, 31, 63, and 127. The C54 (fir) and C55 (fir2) grew at exactly the same rate which implies that the code kernels agree perfectly. The C62 (fir_gen) grew at approximately twice the rate which implies that the library code  is not making full use of the dual multipliers.

Table 1: Real Filter Data

| DSP | Method | N = 15 | N = 31 | N = 63 | N = 127 |
| --- | --- | --- | --- | --- | --- |
| VC5502 | Library | 11 | 19 | 35 | 67 |
|  | PCET | 21 | 29 | 45 | 77 |
| VC5416-160 | Library | 23 | 39 | 71 | 135 |
|  | PCET | 29 | 45 | 77 | 141 |
| 6205-200 | Library | 40 | 72 | 136 | 264 |
|  | PCET | 12 | 24 | 48 | 96 |

Complex Filter

Complex Filters were varied for filter lengths 15, 31, 63, and 127. Only the C54 (cfir) had library code available. The C54 library code required approximately twice as many cycles as the PCET estimate.

Table 2: Complex Filter Data

| DSP | Method | N = 15 | N = 31 | N = 63 | N = 127 |
| --- | --- | --- | --- | --- | --- |
| VC5502 | Library | NA | NA | NA | NA |
|  | PCET | 55 | 87 | 151 | 279 |
| VC5416-160 | Library | 133 | 261 | 517 | 1029 |
|  | PCET | 85 | 149 | 277 | 533 |
| 6205-200 | Library | NA | NA | NA | NA |
|  | PCET | 30 | 59 | 118 | 235 |

LMS Real

Real LMS Filters were varied for lengths 7, 15,  and 31. Only the C54 (dlms) had library code available. The C54 library code required only differed by one cycle from the PCET estimate.

Table 3: LMS Data

| DSP | Method | N = 7 | N = 15 | N = 31 |
| --- | --- | --- | --- | --- |
| VC5502 | Library | NA | NA | NA |
|  | PCET | 21 | 29 | 45 |
| VC5416-160 | Library | 28 | 44 | 76 |
|  | PCET | 29 | 45 | 77 |
| 6205-200 | Library | NA | NA | NA |
|  | PCET | 13 | 26 | 53 |

FFT

Complex FFTs were varied for block lengths 32, 64, 128, and 256. Only the C54 (cfft) and C55 (cfft) had library code available. The C54 library code required a similar number of cycles as estimated by PCET (remarkable given the complexity) while the C55 differed significantly. However, the C55 differs greatly. Oddly, the C55 library code required significantly more cycles than the C54 code, which is surprising as the C55 is the more sophisticated processor. Likely, there are hidden switches in the library code which are adding significantly to the code estimates. Note that the C62 did not support bit-reverse addressing, so no estimates were made.

Table 4: FFT Data

| DSP | Method | N = 32 | N = 64 | N = 256 | N = 1024 |
| --- | --- | --- | --- | --- | --- |
| VC5502 | Library | 517 | 1036 | 4858 | 23848 |
|  | PCET | 406 | 730 | 2594 | 9930 |
| VC5416-160 | Library | 390 | 806 | 3302 | 13286 |
|  | PCET | 431 | 773 | 2737 | 10461 |
| 6205-200 | Library | NA | NA | NA | NA |
|  | PCET | NA | NA | NA | NA |

## External Validation

Matthew Carrick and Shareef Sayed of Virginia Tech recently compared the estimates provided by PCET against profiled measurements from the PowerPC 405D5. To do this, Shareef wrote a DSP file for the PowerPC 405D5 from the available documentation and then used existing PCET components. Efforts were made to translate the documented module assembly into C which was then compiled and executed on the PowerPC 405D5. To measure the length of run time on the 405D5, the processor's 64-bit timer register is accessed before and after running each component 500 times and storing the difference. This allowed for finer grained measurements of the estimated execution time and measurements for components that require less than 1 us to execute. This was performed for 5 components (real filter, complex filter, LMSReal, Taylor, and CORDIC) and the results are shown below. Because this is compiled C code, it would be expected that PCET (which is modeling aggressive hand-coded assembly) would provide smaller estimates. Largely, this is what happened, though the difference varied significantly.

| Component | Length | PCET Cycles | PCET Time | Profiled Time |
| --- | --- | --- | --- | --- |
| RealFilter | 20 | N/A | N/A | 0.41 us |
| ComplexFilter | 10 | N/A | N/A | 0.41 us |
| LMSReal | 10 | 163 | 0.54 us | 8.28 us |
| Taylor | 10 | 86 | 0.29 us | 0.31 us |
| CORDIC | 14 | 289 | 0.96 us | 0.21 us |